Integration Test Scenarios

Simulation + Field Tests

8-Robot Heterogeneous Swarm — 60 Scenarios

문서번호 SAN-TST-INT-001

v1.5

㈜스카이오토넷 / 군집제어형 소형 전술로봇 사업단

| Document ID | SAN-TST-INT-001 |
| --- | --- |
| Version | v1.5 |
| Classification | Test Specification |
| Preceding Docs | SAN-SDD-SWARM-001 v1.5, SAN-IDS-CMD-001 v1.5, SAN-SDD-SUR-001 v1.5 |
| Test Environment | Gazebo Harmonic + ROS 2 Humble (Sim) + Real robot (Field) |
| Scenario Count | 66 (Sim 59 + Real 7) — +6 new S18 scenarios in v1.5 |
| KPP Mapping | 10 KPPs directly measured |

# Revision History

| Version | Changes |
| --- | --- |
| v1.0 | Initial release. 42 scenarios (S1-S14). |
| v1.1 | Added 6 new S15 scenarios (total 48): 360° surveillance, sweep detection, Hub dual SBC, SLAM, GStreamer SRT, multi-video. |
| v1.2 | PM decisions D1~D7 + A1~A5 applied (total 54 scenarios): (D1) 9 -> 8 robots (S15 scenarios adjusted for robot count); (A2) **6 new S16 scenarios** — DEMO 6-phase sequencer verification; (A5) Section 2.5 **bench test environment** added — lightweight CI without Gazebo; Regression CI matrix gains bench tier (PR-level 10 min). |
| v1.3 | Cost Map + LTE Redundancy + SLAM 5s testing added (60 scenarios total): (1) 6 new S17 scenarios — E1 ground filter, 235mm obstacle, 220mm ditch, 30 deg slope, Cost Map KPP, LTE fail-over; (2) S15-4 SLAM consistency — period 30s/60s -> 5s updated; (3) bench env adds Cost Map dummy simulation. |
| v1.4 | Deputy UGV + Limp Mode tests added (66 scenarios total): (1) **6 new S18 scenarios** — Leader->Deputy succession, Leader+Deputy->Hub succession, Hub->Deputy takeover, 3-fail->battery follower, Limp Mode entry/exit; (2) Section 1.2 squadron table updated (Deputy UGV S3 + Followers 1~5). |
| v1.5 | DCN-2026-001 applied (2026-05-12): (1) S18-1~6 scenario bodies formally added (§11, was Revision-History-only in v1.4); (2) §1.2 Total 60 → 66 (Sim 59 + Real 7); (3) §2.3 9-robot test → 8-robot Standard + 4-robot Min variants; (4) §11 Scenario Index renumbered to §12 with header "66 total"; (5) Related Docs version pins updated v1.4 → v1.5; (6) Body references "9-robot V" / "9 robots" → "8-robot" throughout. |

# Table of Contents

# 1. Overview

## 1.1 Document Purpose

This document defines integration test scenarios for the SAN swarm-controlled small tactical robot system. Each scenario provides a standard procedure to verify KPP satisfaction and regression stability under simulation (Gazebo Harmonic) or real-robot environment.

## 1.2 Scenario Classification

| Category | Count | Environment | Purpose |
| --- | --- | --- | --- |
| Basic function | 15 | Gazebo | Verify core functions |
| KPP verification | 12 | Gazebo + real robot | Quantitative measurement of 10 KPPs |
| DEMO sequence | 6 | Gazebo + real robot | DEMO 6-phase auto-demo verification (v1.2) |
| Cost Map + LTE | 6 | Gazebo + bench + real | Cost Map avoidance + LTE redundancy (v1.3 new) |
| Stress | 9 | Gazebo | Comm loss, jamming, multi-obstacle extremes |
| Regression | 12 | Gazebo CI | Auto-verify retained functions |
| Total | 60 | — | — |

## 1.3 KPP Mapping

| KPP | Target | Verification Scenarios | Measurement |
| --- | --- | --- | --- |
| Formation hold avg error | ≤ 2 m | S2-2, S6-2, S12-1, S14-1 | Follower actual vs desired RMSE |
| Proximity reaction | ≤ 300 ms | S9-1, S9-2, S12-2 | Obstacle detect → avoidance start |
| Swarm control latency | ≤ 150 ms | S7-3, S8-2, S12-3 | Message timestamp differences |
| Leader reform | ≤ 10 sec | S10-3, S12-4, S14-2 | Leader OFF → new Leader stabilize |
| Assembly success rate | ≥ 95% | S4-1, S4-2, S12-5, S14-3 | 100 reps success count |
| Drone detection (RF) | ≤ 2 km | S13-1, S13-2 | RF scanner range |
| Real-time video | ≤ 200 ms | S15-5 | GStreamer SRT latency |
| Target classification | ≥ 90% (≤ 2s) | S15-2 | Pan-tilt sweep detection rate |
| Pan-tilt tracking | ≤ 0.05° (500m, 3m/s) | S13-3 | Track mode precision |
| Firing accuracy | ≥ 90% (150m, 45cm) | Live-fire range (separate) | RCWS precision strike |

# 2. Test Environments

## 2.1 Simulation Environment

| Item | Specification |
| --- | --- |
| OS | Ubuntu 22.04 LTS |
| ROS 2 | Humble Hawksbill |
| Simulator | Gazebo Harmonic (Sim 8) |
| UGV model | 330kg tracked (custom URDF + Gazebo plugin) |
| Leader model | Unitree Go2 URDF (champ_description) |
| LiDAR model | Robosense E1 (UGV), Unitree L1 (Go2) — v1.1 update |
| Comm sim | ros2_network_sim (latency/loss injection), OpenWrt mesh sim |
| RTK GNSS | gnss_sim with RTK fixed/float toggle |
| Behavior Tree | BehaviorTree.CPP v4 + Groot2 |
| Test runner | pytest + ros2 launch_testing |
| CI/CD | GitHub Actions + Docker (regression suite) |

## 2.2 Simulation Worlds (5 types)

| World ID | Name | Use | Key Features |
| --- | --- | --- | --- |
| W1 | open_field | Open-area recon | 50x50 m flat, scattered obstacles |
| W2 | forest | Wooded area | 100-300 procedurally generated trees |
| W3 | urban_blocks | Urban alleys | 5x5 blocks, corridor width 2-4m |
| W4 | mountain_dem | Mountain/valley | DEM heightmap, cliff edges |
| W5 | mixed_combat | Combined ops | Open + woods + narrow + cliff |

## 2.3 Real-Robot Environment

| Item | Specification |
| --- | --- |
| Location | Internal test site (Phase 1) + integrated training ground (Phase 2) |
| 3-robot test | Unitree Go2 1 + tracked UGV 2 (Robosense E1) |
| 8-robot test (Standard) + 4-robot test (Minimum) | Unitree Go2 1 + Hub UGV 1 (dual SBC) + tracked Follower UGV 6 (Robosense E1) |
| RTK base | Embedded in operator tablet or base-station deployment |
| Wi-Fi 6 mesh | OpenWrt 24.10, 4-6 APs distributed |
| Weather | Clear + rain/fog partial conditions |
| Live-fire test | Certified range separately, live ammo |

## 2.4 Measurement Tools

- rosbag2 — record all messages

- PlotJuggler — real-time graph analysis

- Foxglove Studio — multi-message synchronized visualization

- ros2_kpp_recorder — automatic KPP measurement (in-house)

- Ground truth: Gazebo /world/model or real-robot external RTK

- GStreamer analysis: ffprobe, GStreamer GST_DEBUG=*:5

- Mesh diagnostics: batctl, mwan3 status, openwrt-stats

## 2.5 bench Test Environment — Lightweight CI (v1.2 new)

Environment for fast regression on internal PC without Gazebo. Runs automatically on every PR with 10-min results.

### 2.5.1 Environment

| Item | Specification |
| --- | --- |
| Environment | Ubuntu 22.04 + ROS 2 Humble (Docker container) |
| Hardware | x86_64 or ARM64 PC, 8GB RAM, no GPU required |
| Dummy node | combat_robot_test_dummy_node — IMU/Pan-Tilt/Detector/Gun sim publish |
| Target deployment_mode | bench (yaml overlay, HW watchdog OFF, dummy data ON) |
| Verification scope | FSM transitions, message communication, swarm relay, deputy fail-over, S16 DEMO sequence |
| Not possible | Real-time video, AI inference (Hailo8), SLAM consistency (Gazebo needed) |

### 2.5.2 CI Tier Hierarchy

| Tier | Frequency | Environment | Time | Pass criteria |
| --- | --- | --- | --- | --- |
| L1 bench | Every PR | Docker + dummy | 10 min | S1, S3, S4, S10, S15-1, S16 PASS |
| L2 Gazebo | Nightly | Gazebo Harmonic | 1-2 hrs | S5-S11, S15-2~6 PASS |
| L3 Real (HIL) | Weekly | Real robot 3 units | 2 hrs | S13 PASS |
| L4 Real full 8 | Monthly/release | Real robot full set | 4 hrs | S14, S15-5 PASS |

# 3. Test Scenarios — Sprint 1-4 (Tracking + Formation)

## 3.1 S1 — Body-Frame Offset PID Tracking

### S1-1 — Stationary Leader Follower Placement

| Attribute | Value |
| --- | --- |
| Purpose | Verify follower reaches desired offset when Leader stationary |
| Environment | W1 open_field |
| Setup | Leader (0,0), Follower (10,5) start, target offset (-5,0) |
| Steps | (1) Leader pose publish → (2) PID tracker enable → (3) 30s observation |
| Pass | Within 30s reach (-5,0) ±0.5m, RMS oscillation <0.2m, overshoot <10% |

### S1-2 — Linear Motion Leader Tracking (3-robot V)

| Attribute | Value |
| --- | --- |
| Purpose | Verify V-formation (d=5m, θ=90°) maintained during Leader linear motion |
| Environment | W1, 50m straight |
| Setup | Leader 1.0 m/s, 2 followers V-formation |
| Pass | 50m run, avg formation error ≤ 2.0m, max ≤ 3.0m |

### S1-3 — Curved Path Tracking (3-robot V)

| Attribute | Value |
| --- | --- |
| Purpose | Verify V hold on curved paths (radius 5m, 10m, 20m) |
| Environment | W1, circular path |
| Pass | Avg error ≤ 2.5m at each radius (5% KPP margin) |

## 3.2 S2 — Hybrid A* Global Path Planning

### S2-1 — Static Obstacle Avoidance Path

| Attribute | Value |
| --- | --- |
| Environment | W2 forest (50x50m, 100 trees) |
| Setup | Leader (0,0), goal (40,40), static OccupancyGrid |
| Pass | Hybrid A* path generated in 5s, 0 collisions, path/straight ratio ≤1.5x |

### S2-2 — Dynamic Replan (Obstacle Added)

| Attribute | Value |
| --- | --- |
| Environment | W2, new obstacle spawn during driving |
| Pass | Replan ≤ 2s, collision avoided, formation hold avg ≤ 2m |

## 3.3 S3 — 1-Second Prediction Broadcast

### S3-1 — FollowerTargetMessage 100ms Period

| Attribute | Value |
| --- | --- |
| Environment | W1, Wi-Fi6 mesh sim (50ms latency + 1% loss) |
| Setup | Leader 1.0 m/s, 3 followers, /swarm/predict/follower_target rosbag |
| Pass | 10Hz ±10% publish, deadline < 200ms, sequence_num monotonic |

## 3.4 S4 — Hungarian Slot Assignment

### S4-1 — V → Column Transition (Assembly Success Rate)

| Attribute | Value |
| --- | --- |
| Environment | W1, 3 robots V start |
| Setup | "Formation transition: column" command 100 repetitions |
| Pass | Assembly success ≥ 95% (KPP), avg transition ≤ 10s |

### S4-2 — Column → Diamond Transition

| Attribute | Value |
| --- | --- |
| Setup | 4-robot column → diamond, 100 reps |
| Pass | Success ≥ 95%, 0 collisions, avg transition ≤ 8s |

# 4. Test Scenarios — Sprint 5-8 (Network + Robustness)

## 4.1 S5 — All 9 Formation Types

### S5-1 — All 9 Formations Entry/Stabilization

| Attribute | Value |
| --- | --- |
| Environment | W1, 8-robot squadron |
| Setup | Each formation 5 entries (45 attempts) |
| Pass | Overall success ≥ 95%, 0 collisions per formation |

### S5-2 — Formation Auto-Transition (Terrain-Aware)

| Attribute | Value |
| --- | --- |
| Environment | W4 mountain (cliff edges) |
| Pass | Auto column entry within 1m of cliff, 5s override window functional |

## 4.2 S6 — V Formation + 8-Robot Stability

### S6-1 — 8-Robot V Linear Drive (5 min)

| Attribute | Value |
| --- | --- |
| Environment | W1, 8-robot V (d=5m, θ=60°) |
| Pass | 5-min run, avg error ≤ 2m, max error ≤ 4m |

### S6-2 — 8-Robot V Curved Drive

| Attribute | Value |
| --- | --- |
| Setup | 10m radius circular path, 8-robot V |
| Pass | Formation hold KPP ≤ 2m met |

## 4.3 S7 — Wi-Fi 6 Mesh + DDS Communication

### S7-1 — Mesh Node Add/Remove Hop

| Attribute | Value |
| --- | --- |
| Environment | OpenWrt mesh sim (3 nodes → 4 nodes → 2 nodes) |
| Pass | On topology change DDS msg loss ≤ 5%, mesh re-converge ≤ 8s |

### S7-2 — Multicast (DDS) Normal Operation

| Attribute | Value |
| --- | --- |
| Setup | OpenWrt multicast_mode=1, igmp_snooping=1 enabled |
| Pass | 8-robot DDS discovery all success, broadcast msg receive 100% |

### S7-3 — KPP Latency ≤ 150ms

| Attribute | Value |
| --- | --- |
| Environment | OpenWrt mesh, 8 robots |
| Measurement | FollowerTarget (10Hz) publish→receive latency, 100,000 samples |
| Pass | p99 ≤ 150ms (KPP), p50 ≤ 50ms |

## 4.4 S8 — LTE Failover (mwan3)

### S8-1 — Auto LTE Switch on Wi-Fi Loss

| Attribute | Value |
| --- | --- |
| Environment | Hub UGV SBC #2 mwan3 enabled |
| Setup | Wi-Fi 5GHz artificially blocked during driving |
| Pass | LTE backhaul switchover within 8s, operator screen info uninterrupted |

### S8-2 — Latency in LTE Environment

| Attribute | Value |
| --- | --- |
| Measurement | LTE backhaul communication latency |
| Pass | Remote C2 command ≤ 200ms (LTE), operator telemetry ≤ 100ms |

# 5. Test Scenarios — Sprint 9-12 (Safety + Election + Threat)

## 5.1 S9 — 5-Tier Autonomous Avoidance

### S9-1 — T1.5 Auto ±2m Detour (KPP 300ms)

| Attribute | Value |
| --- | --- |
| Environment | W2 forest, dynamic obstacle during driving |
| Measurement | Obstacle detect → Tier 1.5 entry → detour start time |
| Pass | ≤ 300ms (KPP), detour success 100%, 0 collisions |

### S9-2 — Tier 2/3 Catch-Up

| Attribute | Value |
| --- | --- |
| Setup | Apply artificial 30% speed limit to 1 follower → induce lag |
| Pass | T2 entry 1.2x speed, T3 entry max speed, recovery transitions T1.5/T1 |

## 5.2 S10 — Modified Raft Leader Election

### S10-1 — Hub UGV Normal Succession

| Attribute | Value |
| --- | --- |
| Environment | W1, 8-robot squadron |
| Setup | Leader Go2 artificially powered off (battery 0%) |
| Pass | Hub UGV succeeds as priority 1, mission continues |

### S10-2 — Quorum Unmet Wait

| Attribute | Value |
| --- | --- |
| Setup | 9 → 4 robots reduced then leader lost |
| Pass | Quorum (5) unmet, 30s wait then retry, "Re-organizing" displayed |

### S10-3 — KPP Reform ≤ 10s

| Attribute | Value |
| --- | --- |
| Measurement | Leader OFF → new leader formation stabilize time (100 reps) |
| Pass | p99 ≤ 10s (KPP), avg ≤ 6s |

## 5.3 S11 — Threat Detection + Fire Control

### S11-1 — Drone Detection (RF Scanner)

| Attribute | Value |
| --- | --- |
| Environment | W5, sim drone spawn at 2km distance |
| Pass | RF scanner detect ≤ 2km (KPP), DetectedObject publish ≤ 100ms |

### S11-2 — Fire Authorization Security

| Attribute | Value |
| --- | --- |
| Setup | Issue fire command without operator PIN |
| Pass | FireAuthorization rejected, audit log recorded, 0 auto-fires |

## 5.4 S12 — KPP Comprehensive

### S12-1 ~ S12-5 — 5 KPP 100 Repetitions (Regression Suite)

Auto-executed in CI. 100 measurements per KPP, p99/p50 distribution analysis.

# 6. Test Scenarios — Sprint 13-14 (Real Robot)

## 6.1 S13 — Real Robot 3 Units (Go2 + 2 UGV)

### S13-1 — Wi-Fi 6 Mesh Outdoor Verification

| Attribute | Value |
| --- | --- |
| Location | Internal test site (open 50x50m) |
| Setup | OpenWrt 24.10 router 3 units, DAWN AP steering, batman-adv |
| Pass | 3-hop mesh operational, throughput ≥ 50 Mbps, latency p99 ≤ 30ms |

### S13-2 — Real Drone Detection (Optional, Safe Env)

| Attribute | Value |
| --- | --- |
| Setup | Civilian FPV drone simulated intrusion (with pre-authorization) |
| Pass | RF scanner detection range, AI classification accuracy ≥ 90% |

### S13-3 — Pan-Tilt Tracking Precision

| Attribute | Value |
| --- | --- |
| Setup | 500m distance target, 3 m/s laterally moving RC vehicle |
| Pass | Tracking error ≤ 0.05° (KPP) |

## 6.2 S14 — Real Robot 9 Units (Go2 + 8 UGV)

### S14-1 — 8-Robot V Outdoor Drive

| Attribute | Value |
| --- | --- |
| Location | Integrated training ground (open + partial woodland) |
| Distance | ~500m mission scenario |
| Pass | Formation hold avg ≤ 2m, comm latency ≤ 150ms |

### S14-2 — Artificial Leader Loss + Hub Succession

| Attribute | Value |
| --- | --- |
| Setup | Leader Go2 powered off → Hub UGV succession |
| Pass | Reform ≤ 10s, mission continues, operator screen shows "Hub Leader" |

### S14-3 — 8-Robot Auto Mission (A→F)

| Attribute | Value |
| --- | --- |
| Setup | "Offensive mode" auto flow — Assembly→Column→V→Assault→Rank→Merge→Column |
| Pass | All stages auto-transition, assembly success ≥ 95%, avg mission ≤ 10 min |

# 7. Test Scenarios — S15 v1.1 New (6 scenarios)

## 7.1 S15-1 — 8-Robot 360° Surveillance Distribution Accuracy

| Attribute | Value |
| --- | --- |
| Scenario ID | S15-1 |
| Environment | Gazebo W5 mixed_combat (sim 8 robots) |
| Purpose | Verify Leader 10s-period sector assignment accuracy |
| Duration | 10 min |

### Setup

- 8-robot V-formation (d=5m, θ=60°, Leader S1 + Hub UGV S2 + F1-F6 = S3-S8), W5 environment

- Leader publishes SurveillanceSectorAssignment (10s period)

- Each Follower runs sweep mode (30°/sec) via PanTiltCommand

- Gazebo /world/model ground truth for all robot positions

### Test Steps

- Verify Leader publishes sector_assign every 10s (rosbag2)

- Each follower performs sweep within assigned sector (Gazebo pan-tilt joints)

- On threat detection event (sim drone spawn), verify immediate reassignment

- On 1 follower lost, verify gap-filling reassignment

- On formation change (V → diamond), verify immediate reassignment

### Pass Criteria

- Periodic refresh: 10s ± 0.5s

- 360° coverage: 8 robots (Leader+F1-6+Hub) summed sectors cover 360° (overlap allowed)

- Sweep operating ratio: ≥ 95%

- Threat detection sector focus response: ≤ 1s

- Follower lost gap-filling response: ≤ 2s

## 7.2 S15-2 — Pan-Tilt Sweep Target Detection Rate

| Attribute | Value |
| --- | --- |
| Scenario ID | S15-2 |
| Environment | Gazebo W2 forest |
| Purpose | Verify KPP ≥ 90% target detection during sweep operation while driving |
| Duration | 15 min |

### Setup

- 8-robot squadron W2 driving, Leader 1.0 m/s

- Sweep mode 30°/sec, HFOV 60°

- 50 Gazebo simulated targets (human silhouettes) randomly spawned in sectors

- Measure latency from target appearance to AI detection

### Pass Criteria

- Target detection rate ≥ 90% (KPP)

- Detection latency ≤ 2s (exposure within one sweep period)

- False positive < 5%

- Sweep → Track mode transition response < 200ms

## 7.3 S15-3 — Hub SBC #1/#2 Failure Partial Operation

| Attribute | Value |
| --- | --- |
| Scenario ID | S15-3 |
| Environment | Gazebo W1 open_field |
| Purpose | Verify partial operation when one of Hub UGV's dual SBCs fails |
| Duration | 20 min |

### Test Cases

#### Case A: SBC #1 (SLAM) Failure

- Setup: Kill Hub UGV SBC #1 process

- Expected: Aggregated SLAM dispatch suspended, each robot continues with local SLAM

- Pass: Comm/video/1s-prediction all normal, operator screen "Global map update suspended" notice

#### Case B: SBC #2 (Comm) Failure

- Setup: Kill Hub UGV SBC #2 process

- Expected: Other Wi-Fi6 mesh robots assist routing, LTE/video suspended

- Pass: Intra-swarm comm continues, operator screen "Hub comm SBC failure" displayed

#### Case C: Auto-Reboot Attempt

- Setup: SBC #2 auto-reboot (1 attempt limit)

- Pass: Reboot complete within 60s, video stream recovered

## 7.4 S15-4 — SLAM 5s Period Global Map Consistency (v1.3 updated)

| Attribute | Value |
| --- | --- |
| Scenario ID | S15-4 |
| Environment | Gazebo W3 urban_blocks |
| Purpose | Verify global map consistency KPP at SLAM 30/60s periods |
| Duration | 20 min |

### Test Cases

#### Case A: 30s Period (default)

- Setup: SLAMLocalDelta 30s period, 8 robots

- Pass: Post-merge grid alignment error ≤ 0.5m (vs Gazebo ground truth)

- Comm load measured ≤ 5 KB/s (target 1-3 KB/s, 100x reduction verified)

#### Case B: 60s Period (Wide-Area Recon)

- Pass: Grid alignment error ≤ 0.8m

#### Case C: 15s Period (Urban Mode)

- Pass: Alignment error ≤ 0.3m, comm load ≤ 10 KB/s

#### Case D: Immediate Update After Formation Change

- Pass: Immediate (1x) SLAMAggregatedMap broadcast post formation change

## 7.5 S15-5 — GStreamer SRT 200ms Latency KPP

| Attribute | Value |
| --- | --- |
| Scenario ID | S15-5 |
| Environment | Real robot (internal test site, LTE environment) |
| Purpose | Verify GStreamer SRT video streaming KPP "real-time ≤ 200ms" |
| Duration | 30 min (repeat) |

### Setup

- GStreamer SRT pipeline enabled on Hub UGV SBC #2

- Operator Android APP issues VideoStreamRequest

- LTE backhaul (Hub → internal LTE modem → Operator)

- Measure end-to-end timestamp from test pattern encoding to decoding display

### Test Steps

- Operate FHD 1.5Mbps H.265 SRT live latency=120ms

- Measure end-to-end latency at Operator APP (10 min, 1Hz)

- Track latency change on LTE signal strength variation (RSSI -90 to -110 dBm)

- Switch to HD 3Mbps and repeat measurement

- Verify SRT ARQ recovery after 5% packet loss injection

### Pass Criteria

- E2E latency: p99 ≤ 200ms (KPP), p50 ≤ 150ms

- Frame loss: ≤ 1% (after SRT ARQ recovery)

- Uninterrupted stream despite LTE signal change (auto bitrate)

- AES-128 encryption active (verify passphrase auto-issuance)

## 7.6 S15-6 — Multi-Follower Simultaneous Video Monitoring

| Attribute | Value |
| --- | --- |
| Scenario ID | S15-6 |
| Environment | Real robot (Operator APP) |
| Purpose | Verify simultaneous multi-follower video + auto quality adjustment |
| Duration | 30 min |

### Test Cases

#### Case A: 1 Stream HD

- Pass: 1920x1080 30fps 3Mbps stable, latency ≤ 200ms

#### Case B: 3 Streams Simultaneous (FHD)

- Setup: F1, F3, Hub video simultaneously requested

- Pass: 3 streams 1280x720 30fps 1.5Mbps stable, total 4.5 Mbps

#### Case C: 4+ Streams → Auto Thumbnail Downgrade

- Setup: 5 follower videos simultaneously requested

- Pass: Auto thumbnail mode (320x240 5fps 100Kbps) entry, total ≤ 500 Kbps

#### Case D: VideoStreamStop Normal Cleanup

- Setup: Operator taps "Stop View Video"

- Pass: GStreamer pipeline cleanup within 5s, follower CPU usage idle

# 8. Regression Test (CI) Matrix

## 8.1 CI Auto-Execution

| Regression Scenario | Frequency | Environment | Runtime | Pass Criteria |
| --- | --- | --- | --- | --- |
| S1-2, S3-1 (bench) | Per PR ★ | bench (dummy) | 5 min | FSM transitions OK, message routing OK |
| S16-1 ~ S16-6 (bench) | Per PR ★ | bench (dummy) | 5 min | DEMO 6-phase completes normally |
| S1-2, S2-2 | Nightly | Gazebo | 10 min | 0 collisions, avg error ≤ 2.5m |
| S4-1, S5-1 | Nightly | Gazebo | 15 min | 10Hz±10%, success ≥ 95% |
| S7-3 | Nightly | Gazebo + netem | 30 min | p99 ≤ 150ms |
| S10-3 | Nightly | Gazebo | 20 min | p99 ≤ 10s |
| S15-1, S15-2 | Weekly | Gazebo W5/W2 | 30 min | Sector accuracy, detection ≥ 90% |
| S15-3, S15-4 | Weekly | Gazebo W1/W3 | 30 min | Partial op + map alignment ≤ 0.5m |
| Full regression suite | Pre-release | Gazebo + real robot | 4 hrs | All KPPs PASS |

★ = v1.2 new bench tier (10-min regression with dummy node, no Gazebo)

## 8.2 Report Format

- rosbag2 (.mcap) — raw message recording

- PlotJuggler analysis (.png + .csv)

- KPP measurement JSON (Jenkins/Grafana visualization)

- Auto Pass/Fail report + Slack alerts

# 9. S16 — DEMO 6-Phase Sequencer Scenarios (v1.2 new)

Per decision A2, six new scenarios verifying the DEMO 6-phase auto-sequencer. For DAPA demos, VIP tours, and tech verification.

## 9.1 S16-1 — DEMO 6-Phase Full Flow (bench)

| Attribute | Value |
| --- | --- |
| Scenario ID | S16-1 |
| Environment | bench (Docker + dummy node) |
| Purpose | Verify full DEMO IDLE -> FORWARD -> SCAN -> ENGAGE -> REVERSE -> COMPLETE pass |
| Test time | 5 min (PR CI) |

### Setup

- deployment_mode = bench (yaml overlay applied)

- combat_robot_test_dummy_node active (IMU/Pan-Tilt/Detector/Gun simulated)

- App issues RECON demo command

### Pass Criteria

- All 6 phases pass normally, auto transition between phases

- Total sequence completes within 60s (default demo_* distance/duration)

- Final IDLE return + COMPLETE displayed

## 9.2 S16-2 — DEMO Stop / Resume

| Attribute | Value |
| --- | --- |
| Scenario ID | S16-2 |
| Environment | bench |
| Purpose | Verify forced stop (resetDemoSequence) and resume behavior during DEMO |

### Test Cases

#### Case A: Stop during SCAN phase

- Setup: Enter SCAN phase, after 5s issue Stop Demo command

- Pass: resetDemoSequence called, immediate IDLE return, actuators stop

#### Case B: EmergencyStop during ENGAGE phase

- Setup: ENGAGE phase fire started, then EmergencyStop issued

- Pass: Fire halts immediately, EMERGENCY_STOP_STATE entered, gun_trigger_permission=DENIED

#### Case C: Resume

- Setup: After stop, Restart Demo command

- Pass: Restarts from FORWARD phase

## 9.3 S16-3 — deployment_mode=demo Weapon Disable Verification

| Attribute | Value |
| --- | --- |
| Scenario ID | S16-3 |
| Environment | bench + Gazebo |
| Purpose | Verify ENGAGE phase does not emit PWM fire signal in demo mode |

### Pass Criteria

- With deployment_mode=demo, gun_trigger_permission=DENIED is forced

- ENGAGE phase: PWM signal stays at 0 (oscilloscope/GPIO measurement)

- Operator screen shows yellow banner "Demo mode — weapons disabled"

- audit log records "deployment_mode=demo, fire suppressed"

## 9.4 S16-4 — DEMO Multi-Robot Synchronized Demo (Leader + Hub)

| Attribute | Value |
| --- | --- |
| Scenario ID | S16-4 |
| Environment | Gazebo W1 or real robot (3-robot minimum) |
| Purpose | Synchronized demo with Leader (S1) + Hub UGV (S2) + 1-2 followers |

### Setup

- Leader S1, Hub UGV S2, Follower S3 (3-robot minimum)

- Leader issues RECON demo command -> swarm_coordinator relays SwarmRobotCommand to S2, S3

- All robots progress through phases in sync

### Pass Criteria

- On phase transition, all 3 robots transit within ±500ms

- FORWARD phase: Hub UGV maintains rear position, Followers maintain V-formation

- SCAN phase: 8-robot sector distribution applies (Leader front, Hub rear, Follower flanks)

## 9.5 S16-5 — DAPA Demo Regression (Real Robot)

| Attribute | Value |
| --- | --- |
| Scenario ID | S16-5 |
| Environment | Real robot integrated training ground (full 8 robots) |
| Purpose | Regression verification for DAPA formal demo scenario |
| Test time | 30 min (3 repetitions) |

### Pass Criteria

- 8-robot simultaneous demo: phase transition sync within ±1s

- Target detection -> ENGAGE auto-transition -> fire (BB pellet/blank)

- Total demo time within 5 min

- All 3 repetitions PASS

## 9.6 S16-6 — command_id Echo Verification (A3)

| Attribute | Value |
| --- | --- |
| Scenario ID | S16-6 |
| Environment | bench (or Gazebo) |
| Purpose | Verify v1.2 new command_id echo pattern |

### Test Cases

#### Case A: FormationCommand command_id echo

- Setup: App issues FormationCommand (command_id=12345)

- Pass: Next RobotStatus message contains last_received_command_id=12345 echo

- All 8 robots echo same command_id (broadcast successful)

#### Case B: Command miss detection

- Setup: Artificial 1% packet loss

- Pass: App detects last_received_command_id not updated within 5s

- App UI displays "Command not received" alert

#### Case C: Sequence verification

- Setup: Issue 100 commands consecutively

- Pass: All 100 command_ids echo received 100%, order preserved

# 10. S17 — Cost Map + LTE Redundancy Scenarios (v1.3 new)

Six new scenarios verifying the Cost Map system (see SDD-SWARM §9.6) and LTE redundancy (§5.5). S17-1~4 are Gazebo simulations, S17-5 is bench (KPP measurement), S17-6 is real robot (LTE failover).

## 10.1 S17-1 — Robosense E1 Ground Plane Segmentation Accuracy

| Attribute | Value |
| --- | --- |
| Scenario ID | S17-1 |
| Environment | Gazebo W2 forest (slopes + grass) |
| Purpose | RANSAC ground segmentation correctly classifies ground points on non-planar terrain |
| Test time | 20 min |

### Pass Criteria

- Ground point classification accuracy >= 95% (vs ground truth)

- False positive (obstacle as ground) < 2%

- Slope 30 deg threshold accurate (30.5 deg = lethal, 29.5 deg = cautious)

## 10.2 S17-2 — 235mm Obstacle Detection + Avoidance

| Attribute | Value |
| --- | --- |
| Scenario ID | S17-2 |
| Environment | Gazebo W1 open_field (obstacle spawn) |
| Purpose | Verify UGV spec lethal threshold 235mm and avoidance behavior |

### Test Cases

- Case A: 200mm brick -> detected as inflated (cost 200), slow pass possible

- Case B: 235mm brick -> lethal (cost 254), mandatory detour

- Case C: 270mm brick -> lethal, detour with +/-2m lateral trajectory

### Pass Criteria

- Case A: pass success, avg speed >= 1.5 m/s

- Case B/C: 0 collisions, 100% detour success

- Tier 1.5 entry time <= 300 ms (existing KPP)

## 10.3 S17-3 — 220mm Ditch Detection + Avoidance

| Attribute | Value |
| --- | --- |
| Scenario ID | S17-3 |
| Environment | Gazebo W4 mountain (ditches/pits) |
| Purpose | Verify UGV spec ditch threshold 220mm and avoidance |

### Test Cases

- Case A: 150mm width ditch -> passable (free)

- Case B: 220mm width ditch -> lethal (exact threshold), mandatory detour

- Case C: 300mm deep, >= 220mm width pit -> lethal + alert

### Pass Criteria

- Case A: pass success

- Case B/C: detour success, 0 collisions/falls

- LiDAR upward scan provides terrain awareness beyond ditches

## 10.4 S17-4 — 30 deg Slope Capability Accurate Verification

| Attribute | Value |
| --- | --- |
| Scenario ID | S17-4 |
| Environment | Gazebo W4 mountain |
| Purpose | Verify UGV spec slope threshold 30 deg is correctly applied |

### Test Cases

- Case A: 25 deg slope -> pass (cautious cost 100, slow down)

- Case B: 30 deg slope (threshold) -> pass attempt (climbing limit, 50% speed)

- Case C: 35 deg slope -> lethal, detour

### Pass Criteria

- Case A/B: pass success, no slipping

- Case C: lethal classification, detour trajectory generated

- Speed during climbing: at 30 deg <= 1.5 m/s (safe slowdown)

## 10.5 S17-5 — Cost Map KPP Verification (latency <= 5s, coverage >= 7m)

| Attribute | Value |
| --- | --- |
| Scenario ID | S17-5 |
| Environment | bench (Docker + Gazebo W1, new KPP measurement) |
| Purpose | v1.3 two new KPPs — cost_map_latency_p99 <= 5s, cost_map_coverage >= 7m |

### Setup

- UGV driving at max speed 12 km/h (3.33 m/s)

- 100 repetitions, p99 latency computed

- Measure /local/cost_map publish time - sensor input time difference

### Pass Criteria

- cost_map_latency_p99 <= 5 sec (measurement target: < 1 sec, 5x margin)

- cost_map_coverage forward >= 7m (14m area published at all times)

- p99 update rate 0.8~1.2 Hz (normal)

- 3 consecutive >5s -> Node Watchdog ERROR_STATE transition confirmed

## 10.6 S17-6 — LTE Redundancy Fail-over (Real Robot)

| Attribute | Value |
| --- | --- |
| Scenario ID | S17-6 |
| Environment | Real robot (Hub S2 + S3 LTE backup + operator) |
| Purpose | §5.5 LTE redundancy — When Hub LTE fails, Follower S3 auto-backup |

### Test Cases

- Case A: Hub LTE modem normal -> S3 in LTE_BACKUP_DESIGNATED (standby)

- Case B: Hub LTE modem artificially disabled -> within 8s S3 LTE_PROMOTED, lte_term += 1

- Case C: After S3 LTE active, verify operator <-> S3 <-> swarm communication

- Case D: Hub LTE recovery -> Hub PROMOTED, S3 DEMOTED, lte_term += 1

### Pass Criteria

- Case B: backup activation time <= 10s (8s mwan3 detect + 2s activation)

- Case C: Operator telemetry uninterrupted (<= 30s temporal gap)

- Case D: No split-brain (only 1 LTE_PROMOTED at any time)

- LTERoleAnnouncement broadcast received normally (all 8 robots)

# 11. Deputy + Limp Mode Scenarios (S18-1 ~ S18-6, ★ v1.5 new — DCN-2026-001 D-006)

Six new scenarios verifying the Hub-Deputy redundancy and 4-tier Leader succession policies defined in SAN-SDD-SWARM-001 v1.5 §5.5 / §5.6 / §5.7. All six are automated by san_l5_regression scenario_runner.cpp and run as part of the L1 bench regression on every PR. Each scenario uses a uniform Setup / Pass Criteria template; deadlines are enforced by corresponding parameters in scenario_runner.hpp (s18_1_deadline_ms .. s18_6_deadline_ms).

## 11.1 S18-1 — Leader → Deputy Succession (1st Priority)

| Attribute | Value |
| --- | --- |
| Scenario ID | S18-1 |
| Environment | bench (Docker dummy injection) + Gazebo W1 |
| Purpose | Verify that when Leader (S1) fails, Deputy UGV (S3) auto-promotes as 1st-priority Leader within ≤ 5 s. |
| Pre-condition | 8-robot baseline seeded (S1 active leader; S3 deputy with SBC1+SBC2 healthy, battery ≥ 20%) |
| Deadline | ≤ 5 s (5000 ms) |
| Implementing test | san_role_management/test/test_leader_succession.cpp, san_l5_regression::runS18_1_LeaderToDeputy() |

### 11.1.1 Setup

• seedEightRobotBaseline(): S1~S8 all sbc1_healthy=true; S2/S3 sbc2_healthy=true; S3 is_deputy_ugv=true; S1 is_leader_role_active=true. Battery 79..72%

• injector_->removeRobot(1): S1 RobotStatus publish stops → LeaderRoleManager watchdog times out after 1.4 s

### 11.1.2 Pass Criteria

• Receive on /swarm/leader/role_announce within ≤ 5 s with: role == LEADER_PROMOTED

• robot_id == 3 (Deputy UGV)

• succession_priority == PRIORITY_DEPUTY (= 1)

• leader_term incremented by ≥ 1 from previous value

• No other robot publishes PROMOTED in the same window (split-brain absent)

## 11.2 S18-2 — Leader + Deputy both fail → Hub Succession (2nd Priority)

| Attribute | Value |
| --- | --- |
| Scenario ID | S18-2 |
| Environment | bench + Gazebo W1 |
| Purpose | Verify that when both Leader (S1) and Deputy (S3) fail, Hub UGV (S2) auto-promotes as 2nd-priority Leader within ≤ 8 s. |
| Pre-condition | 8-robot baseline seeded |
| Deadline | ≤ 8 s (8000 ms) |
| Implementing test | san_l5_regression::runS18_2_LeaderDeputyToHub() |

### 11.2.1 Setup

• Baseline seeded

• removeRobot(1) + removeRobot(3): S1 (Leader) and S3 (Deputy) stopped simultaneously

• S2 (Hub) sbc1/sbc2 both healthy, battery ≥ 20%

### 11.2.2 Pass Criteria

• Within ≤ 8 s receive LeaderRoleAnnouncement(role=LEADER_PROMOTED, robot_id=2, succession_priority=HUB)

• leader_term incremented

• 8 s budget breakdown: Deputy grace 200 ms + Hub grace 400 ms + Deputy timeout 5 s + safety margin

## 11.3 S18-3 — Hub fails → Deputy Full Takeover (LTE + SLAM + Video)

| Attribute | Value |
| --- | --- |
| Scenario ID | S18-3 |
| Environment | bench + Gazebo W1 + real-robot Phase 2 |
| Purpose | Verify that when Hub UGV (S2) fails, Deputy (S3) takes over LTE gateway + SLAM aggregation + GStreamer SRT relay ALL THREE within ≤ 7 s (partial takeover not allowed). |
| Pre-condition | 8-robot baseline; Leader (S1) remains healthy (no Leader succession in this scenario) |
| Deadline | ≤ 7 s (7000 ms) |
| Implementing test | san_role_management/test/test_hub_deputy_takeover.cpp, san_l5_regression::runS18_3_HubToDeputyTakeover() |

### 11.3.1 Setup

• Baseline seeded

• removeRobot(2): Hub UGV total failure (Leader unaffected)

### 11.3.2 Pass Criteria

• Within ≤ 7 s receive HubRoleAnnouncement(role=HUB_PROMOTED, robot_id=3)

• ALL three capability flags simultaneously true:

•   - lte_active == true

•   - slam_aggregation_active == true

•   - video_relay_active == true

• hub_term incremented by ≥ 1

• LTE-only or SLAM-only partial takeover → FAIL (covered separately by §5.14 LTERoleAnnouncement tests)

## 11.4 S18-4 — Leader + Hub + Deputy all fail → Battery-max Follower (3rd Priority)

| Attribute | Value |
| --- | --- |
| Scenario ID | S18-4 |
| Environment | bench + Gazebo W1 |
| Purpose | Verify that when all three priority-1/2 candidates fail, the battery-max follower auto-promotes as 3rd-priority Leader within ≤ 10 s. |
| Pre-condition | 8-robot baseline; battery deliberately distributed S4=60%, S5=90%, S6=45%, S7=55%, S8=30% (S5 is unique winner) |
| Deadline | ≤ 10 s (10000 ms) |
| Implementing test | san_role_management/test/test_battery_selection.cpp, san_l5_regression::runS18_4_ThreeFailedBatteryFollower() |

### 11.4.1 Setup

• Baseline seeded with battery distribution above

• removeRobot(1) + removeRobot(2) + removeRobot(3): Leader + Hub + Deputy all stopped

### 11.4.2 Pass Criteria

• Within ≤ 10 s receive LeaderRoleAnnouncement(role=LEADER_PROMOTED, succession_priority=BATTERY_MAX)

• Promoted robot_id == 5 (S5, battery 90%, unique max)

• Tie-breaker rule (equal battery → lowest robot_id wins) covered by separate test

• 10 s budget breakdown: Deputy/Hub grace + 5 s timeout + 200 ms BATTERY_MAX grace + accumulation

## 11.5 S18-5 — Hub + Deputy both fail → Limp Mode Entry

| Attribute | Value |
| --- | --- |
| Scenario ID | S18-5 |
| Environment | bench + Gazebo W1 |
| Purpose | Verify that when both Hub (S2) and Deputy (S3) fail simultaneously, Limp Mode entry alert is published within ≤ 8 s. (Leader S1 remains healthy.) |
| Pre-condition | 8-robot baseline; Leader (S1) healthy |
| Deadline | ≤ 8 s (8000 ms) |
| Implementing test | san_role_management/test/test_limp_mode.cpp, san_l5_regression::runS18_5_HubDeputyBothDownLimpEnter() |

### 11.5.1 Setup

• Baseline seeded

• removeRobot(2) + removeRobot(3): S2 (Hub) and S3 (Deputy) stopped simultaneously

### 11.5.2 Pass Criteria

• Within ≤ 8 s receive message on /swarm/limp_mode/alert with data containing "LIMP_MODE_ACTIVE"

• Additional verification (covered by integration tests):

•   - Wi-Fi 6 mesh routing reconverges excluding Hub/Deputy nodes

•   - Android app switches to mesh-direct mode via nearest surviving Follower

•   - Fire/strike command interface remains ENABLED (D-004 Option A verification)

•   - GStreamer video sends directly from each robot to Android IP (no Hub relay)

## 11.6 S18-6 — Deputy Recovers → Limp Mode Exit

| Attribute | Value |
| --- | --- |
| Scenario ID | S18-6 |
| Environment | bench + Gazebo W1 |
| Purpose | Verify that after Limp Mode is entered (S18-5), Deputy (S3) recovery triggers Limp Mode exit alert + Hub-role takeover within ≤ 5 s. |
| Pre-condition | System in Limp Mode (S2 + S3 both down, Leader S1 still healthy) |
| Deadline | ≤ 5 s (5000 ms) |
| Implementing test | san_l5_regression::runS18_6_DeputyRecoversLimpExit() |

### 11.6.1 Setup

• Enter Limp Mode per S18-5 (S2 + S3 down) and verify alert received

• Reset alert watcher

• restoreRobot(3, {sbc1_healthy=true, sbc2_healthy=true, battery_percent=75.0, is_deputy_ugv=true}): Deputy comes back online

### 11.6.2 Pass Criteria

• Within ≤ 5 s receive /swarm/limp_mode/alert containing "LIMP_MODE_EXITED"

• Within ≤ 5 s also receive HubRoleAnnouncement(role=HUB_PROMOTED, robot_id=3) with all three flags true

• Hub (S2) recovery in a subsequent step would trigger HUB_DEMOTED (covered by S18-3 reverse test)

## 11.7 S18 Scenario Summary Matrix

| Scenario | Trigger | Expected Succession | Deadline | Implementing Function |
| --- | --- | --- | --- | --- |
| S18-1 | Leader (S1) down | Deputy (S3) → Leader | ≤ 5 s | runS18_1_LeaderToDeputy() |
| S18-2 | Leader + Deputy down | Hub (S2) → Leader | ≤ 8 s | runS18_2_LeaderDeputyToHub() |
| S18-3 | Hub (S2) down | Deputy (S3) → Hub (full) | ≤ 7 s | runS18_3_HubToDeputyTakeover() |
| S18-4 | Leader + Hub + Deputy down | Battery-max follower (S5) → Leader | ≤ 10 s | runS18_4_ThreeFailedBatteryFollower() |
| S18-5 | Hub + Deputy down | LIMP_MODE_ACTIVE alert | ≤ 8 s | runS18_5_HubDeputyBothDownLimpEnter() |
| S18-6 | Deputy recovers from Limp | LIMP_MODE_EXITED + Deputy → Hub | ≤ 5 s | runS18_6_DeputyRecoversLimpExit() |

# 12. Scenario Index (66 total)

## 11.1 Simulation + bench (54)

| # | ID | Theme | Env | KPP |
| --- | --- | --- | --- | --- |
| 1 | S1-1 | Stationary leader tracking | W1 | — |
| 2 | S1-2 | Linear V tracking | W1 | form ≤2m |
| 3 | S1-3 | Curved V tracking | W1 | form ≤2m |
| 4 | S2-1 | Hybrid A* static | W2 | — |
| 5 | S2-2 | Hybrid A* dynamic | W2 | form ≤2m |
| 6 | S3-1 | 10Hz prediction broadcast | W1 | comm ≤150ms |
| 7 | S4-1 | V→column transition | W1 | assemble ≥95% |
| 8 | S4-2 | Column→diamond | W1 | assemble ≥95% |
| 9-13 | S5-1~5 | 9 formations verification | W1, W4 | — |
| 14-15 | S6-1~2 | 9-robot V stability | W1 | form ≤2m |
| 16-18 | S7-1~3 | OpenWrt Mesh + DDS | Sim | comm ≤150ms |
| 19-20 | S8-1~2 | LTE Failover | Sim | — |
| 21-22 | S9-1~2 | 5-Tier autonomous avoidance | W2 | react ≤300ms |
| 23-25 | S10-1~3 | Modified Raft Election | W1 | reform ≤10s |
| 26-27 | S11-1~2 | Threat detect + fire control | W5 | detect ≤2km |
| 28-32 | S12-1~5 | KPP comprehensive (regression) | W1 | 5 KPPs |
| 33-35 | S13-1~3 | Real robot 3 | Internal | — |
| 36-38 | S14-1~3 | Real robot 9 | Range | 5 KPPs |
| 39 | S15-1 | * 360° surveillance | W5 | — |
| 40 | S15-2 | * Pan-tilt sweep detect | W2 | classify ≥90% |
| 41 | S15-3 | * Hub dual SBC partial op | W1 | — |
| 42 | S15-4 | * SLAM 30/60s consistency | W3 | — |
| 43 | S16-1 | ** DEMO 6-Phase full flow | bench | — |
| 44 | S16-2 | ** DEMO Stop/Resume | bench | — |
| 45 | S16-3 | ** DEMO Weapon disable verify | bench+Gazebo | — |
| 46 | S16-4 | ** DEMO Multi-robot sync | Gazebo | — |
| 47 | S16-6 | ** command_id Echo | bench | — |
| 48 | S17-1 | *** E1 ground segmentation | W2 | — |
| 49 | S17-2 | *** 235mm obstacle detect/avoid | W1 | react <=300ms |
| 50 | S17-3 | *** 220mm ditch detect/avoid | W4 | — |
| 51 | S17-4 | *** 30 deg slope verification | W4 | — |
| 52 | S17-5 | *** Cost Map KPP | bench | lat <=5s, cov >=7m |
| 53 | S15-4 | * SLAM 5s consistency (updated) | W3 | — |

## 11.2 Real Robot (6)

| # | ID | Theme | Env | KPP |
| --- | --- | --- | --- | --- |
| 43 | S13-1 | Wi-Fi6 mesh outdoor | Internal site | comm ≤150ms |
| 44 | S13-2 | Real drone detect | Internal site (optional) | detect ≤2km |
| 45 | S13-3 | Pan-tilt track | Internal site | track ≤0.05° |
| 46 | S14-1 | 9-robot V outdoor | Range | form ≤2m |
| 47 | S14-2 | Leader loss + succession | Range | reform ≤10s |
| 48 | S15-5 | * GStreamer SRT 200ms | Internal LTE | video ≤200ms |
| 49 | S16-5 | ** DAPA Demo Regression | Training ground (full 8) | — |
| 50 | S17-6 | *** LTE Redundancy fail-over | Real robot (Hub + S3) | <=10s backup activation |

* = v1.1 new, ** = v1.2 new (DEMO 6-phase), *** = v1.3 new (Cost Map + LTE)