# SkyHunter v1.5.2 — SW 시스템 동작 문서

버전: 1.0 (Initial — DCN-2026-006 EXT + DCN-2026-011 통합판) 대상: 개발자 / 통합 시험 담당자 / 운영자 (시스템 동작 이해 위해) 작성: 김태근 (PM, ㈜스카이오토넷) 최종 수정: 2026-05-14

## 0. Executive Summary

SkyHunter 군집 로봇 시스템은 5개 robot (Leader Go2 + Hub UGV [SBC #1 + #2] + Deputy UGV + Follower UGV × 5) 가 동일한 source tree 와 동일한 colcon build artifact 로 동작합니다. 역할 구별은 launch parameter 만으로 합니다 (소스 코드 분리 없음).

### 핵심 SW Architecture Choice

| 결정 | 선택 | 이유 |
| --- | --- | --- |
| Source tree | Single tree | ROS 2 best practice (Nav2, Autoware 동일 패턴), 코드 중복 0%, 단일 git history |
| Build artifact | Single build | 모든 robot 에 동일 image, Limp Mode 시 hot promotion 가능 |
| Role 구별 | Launch parameter (robot_role, sbc_id 등) | systemd 가 부팅 시 적절한 wrapper 호출 |
| Hardware 차이 | CMake conditional compile (HAVE_RKNN, HAVE_HAILORT 등) | 동일 build 안에서 hardware-specific code 활성/비활성 |
| 통신 | CycloneDDS + WiFi 6 mesh + LTE redundancy | hybrid 통신, 군집 자율 fail-over |

### v1.5.2 패치 통합 결과

12 DCN-2026-006 EXT sub-changes + 4 DCN-2026-011 sub-changes 통합 시: - 41 files, +2,529 / −146 LOC, 56 new tests - 새 토픽 5개: /swarm/threat_alert_raw, /rtk_gnss_node/heading, /diagnostics/robot_status_audit, /diagnostics/hub_slam_audit, /hub_internal/sbc{1,2}/heartbeat - Hub-only gate (P0 hotfix): 모든 robot 에서 instantiate 되지만 발행은 Hub 에서만

## 1. System Architecture Overview

### 1.1 5-Robot 군집 구성

┌────────────────────────────────────────────────────────────────────┐
│                     SkyHunter Squadron (Demo Day)                   │
├────────────────────────────────────────────────────────────────────┤
│                                                                      │
│  Leader Go2 (robot_id=1)     Hub UGV (robot_id=2)                  │
│   ┌──────────┐                ┌──────────┐ ┌──────────┐            │
│   │ RK3588   │                │ SBC #1   │ │ SBC #2   │            │
│   │ Unitree  │                │ RK3588   │ │ RK3588   │            │
│   │ + 무장   │◀──── WiFi6 ────│ (active) │ │(standby) │            │
│   └──────────┘    mesh        └──────────┘ └──────────┘            │
│        ▲                            │            │                 │
│        │                            │ heartbeat ─┘                 │
│        │                            ▼                              │
│   ┌──────────┐                ┌──────────┐    ┌──────────┐         │
│   │Deputy UGV│ ──────────────▶│Follower  │... │Follower  │         │
│   │(id=3)    │  Wi-Fi mesh    │UGV (id=4)│    │UGV (id=8)│         │
│   │ RK3588   │                │ RK3588   │    │ RK3588   │         │
│   └──────────┘                └──────────┘    └──────────┘         │
│                                                                      │
│                  ━━━━ LTE redundancy (mwan3) ━━━━                  │
│                  ━━━━ Wi-Fi 6 mesh (low latency) ━━━━              │
│                  ━━━━ LoRa (emergency only) ━━━━                   │
└────────────────────────────────────────────────────────────────────┘
                              ▲
                              │ WiFi router
                              ▼
                  ┌──────────────────────┐
                  │ Aban Android tablet  │
                  │ (operator UI)         │
                  └──────────────────────┘

### 1.2 SBC 당 SW 스택

각 RK3588 보드의 SW 구성:

┌─────────────────────────────────────────────────────┐
│                User-space (rclcpp)                   │
│  ┌─────────────────────────────────────────────┐    │
│  │ ROS 2 Humble 노드들 (squadron.launch.py)    │    │
│  │                                              │    │
│  │  ┌─Tier 1 (always-on)──────────────────┐   │    │
│  │  │ operation_control_node              │   │    │
│  │  │ san_localization (EKF)              │   │    │
│  │  │ san_nav2                            │   │    │
│  │  │ san_lidar / san_rtk_gnss            │   │    │
│  │  │ role_management_node (3 managers)   │   │    │
│  │  └──────────────────────────────────────┘   │    │
│  │                                              │    │
│  │  ┌─Tier 2 (role-conditional)────────────┐  │    │
│  │  │ hub_only: san_hub_slam, gst_relay,   │  │    │
│  │  │           swarm_coordinator,         │  │    │
│  │  │           threat_aggregator          │  │    │
│  │  │ leader_only: unitree_driver          │  │    │
│  │  │ follower_only: san_follower_tier     │  │    │
│  │  └──────────────────────────────────────┘  │    │
│  └─────────────────────────────────────────────┘    │
├─────────────────────────────────────────────────────┤
│           Middleware (CycloneDDS)                    │
├─────────────────────────────────────────────────────┤
│  systemd (skyautonet-<role>.service - D-035)         │
├─────────────────────────────────────────────────────┤
│  Ubuntu 22.04 LTS (RK3588 BSP)                       │
└─────────────────────────────────────────────────────┘

### 1.3 Boot Sequence

1. RK3588 전원 ON
   └─→ U-Boot → kernel boot
   
2. systemd 시작
   └─→ network-online.target 도달
   
3. systemd 가 skyautonet-<role>.service 시작 (DCN-2026-011 D-035)
   ├─→ Environment 로드 (/etc/san/squadron.env)
   ├─→ ros2 launch san_bringup <role>.launch.py 실행
   │     ├─ leader_go2.launch.py    (Leader Go2)
   │     ├─ hub_sbc1.launch.py      (Hub SBC #1)
   │     ├─ hub_sbc2.launch.py      (Hub SBC #2)
   │     ├─ deputy.launch.py        (Deputy UGV)
   │     └─ follower.launch.py      (Follower UGV)
   │
4. Launch wrapper 가 squadron.launch.py include + hardcoded params
   └─→ robot_id, robot_role, sbc_id, hub_features, deployment_mode 설정
   
5. squadron.launch.py 노드 spawn
   ├─→ Tier 1 always-on (모든 robot)
   ├─→ hub_only nodes (robot_role=='hub' 일 때만)
   ├─→ leader_only nodes (robot_role=='leader' 일 때만)
   └─→ follower_only nodes (robot_role=='follower' 일 때만)

6. 정상 운행 (squadron 가 publish/subscribe 시작)
   └─→ 운영자 Android UI 가 detection

Boot 시간 측정 (Gate-1 통과 기준): ≤ 90 sec from power-on to all nodes alive.

## 2. Role-Specific Configuration Matrix

| 항목 | Leader Go2 | Hub SBC#1 | Hub SBC#2 | Deputy | Follower |
| --- | --- | --- | --- | --- | --- |
| Launch wrapper (D-034) | leader_go2.launch.py | hub_sbc1.launch.py | hub_sbc2.launch.py | deputy.launch.py | follower.launch.py |
| systemd unit (D-035) | skyautonet-leader-go2.service | skyautonet-hub-sbc1.service | skyautonet-hub-sbc2.service | skyautonet-deputy.service | skyautonet-follower.service |
| robot_id | 1 | 2 | 2 | 3 | 4..8 |
| robot_role | leader | hub | hub | deputy | follower |
| sbc_id (D-032) | 0 | 1 | 2 | 0 | 0 |
| hub_features | false | true | true | false | false |
| /etc/skyautonet/sbc_id | “0” | “1” | “2” | “0” | “0” |
| Hub-only 발행 | ✗ | ✓ active | ✓ standby (lifecycle inactive) | ✓ standby | ✗ |
| Heartbeat pub/sub | ✗ | sbc1 ↔ sbc2 | sbc1 ↔ sbc2 | ✗ | ✗ |

### 모든 robot 에서 항상 시작되는 노드들 (Tier 1)

| 노드 | 모듈 | 동작 |
| --- | --- | --- |
| operation_control_node | san_operation_control | RobotStatus 5Hz publish, sbc1/sbc2 fields (D-033) |
| role_management_node | san_role_management | HubRoleManager + LeaderRoleManager + LimpModeManager 모두 인스턴스화 (active vs standby 는 robot_role 로 구분) |
| san_localization (EKF) | robot_localization | imu0 (chassis IMU) + imu1 (RTK heading, D-022) |
| san_nav2 | (Nav2 stack) | path planning |
| human_detector | human_detector | YOLO + RKNN (D-018/019 적용) |
| san_lidar/robosense_e1_driver | san_lidar | LiDAR 처리, RANSAC 실패 시 ThreatAlert (D-017) |
| san_rtk_gnss/rtk_gnss_node | san_rtk_gnss | GGA + HDT parser, /heading publisher (D-022) |
| san_lte_redundancy/lte_link_quality_node | san_lte_redundancy | hysteresis grader (D-020) |

## 3. Topic Graph + 데이터 흐름

### 3.1 Per-Robot 토픽 (모든 robot 발행)

/swarm/robot_<id>/status            (RobotStatus, 5Hz, reliable)
                                     ↑ operation_control_node
                                     ↑ sbc1/sbc2_healthy (D-033)
                                     
/rtk_gnss_node/heading              (Imu yaw-only, 5-10Hz, sensor_data)
                                     ↑ rtk_gnss_node (D-022)
                                     ↑ orientation_covariance[8] varies (D-023)
                                     
/diagnostics                        (DiagnosticArray, 1Hz, reliable)
                                     ↑ robosense_e1_driver (D-017)
                                     ↑ ground segmenter health
                                     
/swarm/threat_alert_raw             (ThreatAlert, event-driven, reliable)
                                     ↑ robosense_e1_driver (D-017, sensor fail)
                                     ↑ rate limited 500ms

### 3.2 Hub-only 토픽 (★ P0 hotfix 적용 — Hub UGV 만 발행)

/diagnostics/robot_status_audit     (DiagnosticArray, 1Hz, reliable)
                                     ↑ HubRoleManager (D-024 + P0 hotfix)
                                     ↑ ONLY published when robot_role=='hub'
                                     ↑ Per-robot rate / freshness / sbc health
                                     
/diagnostics/hub_slam_audit         (DiagnosticArray, ~0.2Hz)
                                     ↑ hub_slam_node (D-026)
                                     ↑ mismatch_ratio, contributing_cells
                                     ↑ severity: ≥10%=ERROR, ≥5%=WARN
                                     
/global/slam_aggregated             (SLAMAggregatedMap, ~0.2Hz, transient_local)
                                     ↑ hub_slam_node
                                     ↑ Bayesian voting (D-021)
                                     
/hub/threat_alert                   (ThreatAlert, dedup window 3s)
                                     ↑ threat_aggregator
                                     
/swarm/formation/follower_target    (FollowerTargetMessage, 10Hz, depth=50)
                                     ↑ swarm_coordinator
                                     ↑ QoS depth 20→50 (D-025)

### 3.3 Hub-internal 토픽 (Hub UGV 내부, SBC #1 ↔ SBC #2)

/hub_internal/sbc1/heartbeat        (Header, 5Hz, best_effort)
                                     ↑ operation_control_node (sbc_id=1)
                                     ↓ operation_control_node (sbc_id=2) subscribes
                                     
/hub_internal/sbc2/heartbeat        (Header, 5Hz, best_effort)
                                     ↑ operation_control_node (sbc_id=2)
                                     ↓ operation_control_node (sbc_id=1) subscribes

→ peerSbcHealthy() = (now - last_peer_heartbeat) < 3.0 sec
→ RobotStatus.sbc1_healthy / sbc2_healthy 채움

### 3.4 데이터 흐름 — Cross-robot (시나리오 A 예시)

Leader Go2:
  human_detector → /detections (Leader's local)
  rtk_gnss_node → /rtk_gnss_node/heading (Leader's RTK)
  operation_control_node → /swarm/robot_1/status
        │
        ▼
Hub UGV (SBC #1, active):
  swarm_coordinator subscribes /swarm/robot_*/status (1..8)
  human_detector × 4 robots' /detections → threat_aggregator
  hub_slam_node aggregates 4 robots' /robot_*/slam_local_delta
        │
        ▼
  swarm_coordinator publishes:
    /swarm/formation/follower_target  (10Hz)
        │
        ▼
Follower UGV (id=4):
  san_follower_tier subscribes /swarm/formation/follower_target
  san_nav2 uses target → cmd_vel
  EKF imu0 (chassis) + imu1 (RTK heading from local rtk_gnss_node)
  → /odometry/filtered/global → san_nav2

## 4. 시나리오 별 SW 동작 상세

### 4.1 Scenario A (Attack Mode) — SW 동작

활성 노드 (Hub UGV SBC #1 기준):

Tier 1 (always-on):    8 nodes 모두 active
Hub-only:              4 nodes active (slam_aggregator, swarm_coordinator,
                                       threat_aggregator, gstreamer_relay)
Leader-only:           N/A (Hub 이므로)
Follower-only:         N/A

KPP-1 (50m+ follower 횡오차 ≤ 2m) 달성 흐름:

T+0s   Follower (id=4) static start
T+5s   IMU drift 시작 (1-3°/min)
T+10s  RTK FIX 획득 → /rtk_gnss_node/heading 발행 시작
       EKF imu1 fuse — orientation_covariance[8] = 0.0017 rad² (D-023)
T+15s  yaw 절대 reference 안정
T+60s  follower 50m+ 거리에서 lateral error < 2m ★ KPP-1 통과

### 4.2 Scenario B (Defense) — Hub SLAM 통합

Bayesian voting flow (D-021):

4 robots 각자 5초마다 /robot_{1..8}/slam_local_delta publish
        │
        ▼
hub_slam_node (Hub-only):
  MultirobotAggregator.applyDeltaRaw(robot_id, grid)
    └─ For each non-UNKNOWN cell:
       free_votes_[i]++   OR   occupied_votes_[i]++
       (saturating at UINT16_MAX)
  
  Periodically (5s timer):
    recomputeGlobal():
      For each cell:
        if (free > occupied)  global[i] = FREE
        elif (occupied > free) global[i] = OCCUPIED
        elif (tie)            keep previous
        else                  UNKNOWN
      tally mismatch_cells (both > 0)
  
  Publish /global/slam_aggregated
  Publish /diagnostics/hub_slam_audit (D-026):
    contributing_robots: 4
    contributing_cells: 12345
    mismatch_cells:     234
    mismatch_ratio:     0.019  (1.9%, < 5% threshold → OK)

### 4.3 Scenario C (Drone Hardkill) — Fire Authorization

Two-key authorization flow (변경 없음, v1.3 유지):

T+0s   /detections → drone detected (confidence 98%)
T+1s   swarm_coordinator → sniper formation 명령
T+30s  san_fire_authorization 가 FireSolutionMessage 계산
T+40s  Android App 가 "PM 인증 요청" 표시
T+45s  PM 의 HMAC 입력 → /fire_auth/pm_key
T+50s  Android App 가 "운영자 2차 입력" 표시
T+55s  운영자의 HMAC 입력 → /fire_auth/operator_key
T+56s  san_fire_authorization: ARMED 상태
       fsync audit log to disk (R-4 audit)
T+58s  Sim fire command 발행
T+60s  Hit visualization

### 4.4 Scenario D (LTE Redundancy) — Hub Failover + Bitrate Ramp

Hub failover (Limp Mode):

T+0s   SBC #1 의 LTE 차단 (mwan3 정책)
T+1s   SBC #1 의 lte_link_quality_node:
         grade GOOD → FAIR (D-020 hysteresis: 즉시 downgrade)
T+3s   grade FAIR → POOR
T+5s   SBC #1 의 hub_role_manager: comm degraded 감지
       SBC #2 의 hub_role_manager: SBC #1 의 heartbeat (D-033) stale
       SBC #2 promote 시작 (hub_term++ for split-brain prevention)
T+10s  SBC #2 가 active Hub:
         - swarm_coordinator (formerly SBC #1) lifecycle deactivate
         - SBC #2 의 swarm_coordinator lifecycle activate
         - /diagnostics/robot_status_audit 발행 주체 변경
         - /swarm/formation/follower_target 발행 주체 변경
       Android UI: "Takeover: 10s ✓"

Bitrate ramp (PR-7) — LTE 복구 시:

T+0s    LTE-1 복구, /lte_link_quality grade = GOOD
T+0s    AutoRateController.onLinkQuality(GOOD)
        hysteresis 통과 (POOR → GOOD = multi-step, allowed)
T+0s    applier callback: applyAutoRateTarget(QUALITY_FHD)
        Jump detect: THUMBNAIL → FHD = 3 levels
        → Ramp 시작 (PR-7)
T+0s    Step 1: applyQualityNow(LOW)
        GStreamer pipeline rebuild × N streams
T+1.5s  Step 2: applyQualityNow(HD)
T+3.0s  Step 3: applyQualityNow(FHD)
T+4.5s  Ramp complete

## 5. 핵심 SW 동작 — 동시성 + 안전성

### 5.1 mutex 보호 영역 (DCN-2026-003 D-005 + DCN-2026-004 D-006)

hub_slam_node::mutex_:
  - aggregator_.applyDelta() — incoming SLAM deltas
  - aggregator_.snapshot()   — publish 시 read
  
  ★ R-3 패턴 (Phase 7 deferred → DCN-2026-006 EXT): 
    snapshot() 만 mutex 안에서 호출 (lightweight memcpy).
    encodePng() 는 mutex 밖에서 호출 (CPU-heavy OpenCV imencode).
    → applyDelta() 가 publish 시간 동안 차단되지 않음.

gstreamer_relay_node::cb_group_:
  MutuallyExclusive callback group 으로 onRequest + onLinkQuality 직렬화
  (DCN-2026-003 D-005 C-1 fix)

### 5.2 Atomic state (lock-free)

ground_segmenter:
  atomic<uint64_t> fail_count_{0}
  atomic<uint64_t> success_count_{0}
  atomic<GroundSegmenterFailReason> last_fail_reason_{NONE}
  (D-017 — single-writer, multi-reader from publishers)

rtk_gnss_node:
  atomic<uint8_t> last_fix_type_{0}
  (D-022 — publishFix sets, publishHeading reads;
   같은 reader thread 에서 sequential 호출 — 향후 thread split 안전성)

### 5.3 Lifecycle management (Hub failover)

hub_role_manager 가 lifecycle clients 보유:
  - /hub_slam_node/change_state
  - /swarm_coordinator/change_state
  - /threat_aggregator/change_state
  - /gstreamer_relay/change_state

Hub promotion 시:
  → 4 lifecycle services 에 ACTIVATE 호출
  → 모든 hub-only 노드가 동시 활성화

Hub demotion 시 (다른 SBC 가 promote 했을 때):
  → DEACTIVATE 호출 → 노드들 cleanup

## 6. SW 진단 + 모니터링 (운영자 시야)

### 6.1 Android App 이 subscribe 하는 토픽

| 토픽 | 화면 | 패치 |
| --- | --- | --- |
| /swarm/robot_*/status | Main / Formation | D-033 sbc_healthy |
| /diagnostics/robot_status_audit | Diagnostics | D-024 (Hub-only) |
| /diagnostics/hub_slam_audit | Diagnostics | D-026 |
| /diagnostics | 시스템 알림 | D-017 LiDAR fail |
| /swarm/threat_alert_raw | Threats | D-017 |
| /hub/threat_alert | Threats | (기존) |
| /lte_link_quality | Comm | D-020 hysteresis |

### 6.2 운영자 알람 — Severity 기준

| Severity | 의미 | Trigger 예시 |
| --- | --- | --- |
| OK | 정상 운행 | mismatch_ratio < 5% |
| WARN | 주의 | LTE FAIR, SLAM mismatch_ratio 5-10%, RobotStatus rate < 2.5Hz |
| ERROR | 즉시 조치 | RobotStatus 0 samples in window, SLAM mismatch_ratio ≥ 10% |
| STALE | 통신 끊김 | stale_ms ≥ 3000 (3초 이상 RobotStatus 없음) |

## 7. SW 변경 사항 통합 매트릭스

### 7.1 DCN-2026-006 EXT (12 sub-changes)

| DCN | 모듈 | 변경 사항 | KPP/Gate |
| --- | --- | --- | --- |
| D-015 | san_costmap | cellH 경계 boundary nullopt | Gate-1 (5km/h) |
| D-016 | san_costmap | ditch detection 가 obstacle shadow 구분 | Gate-1 |
| D-017 | san_lidar | RANSAC fail → ThreatAlert publish | Gate-1 |
| D-018 | human_detector | fastSigmoid NaN guard + [-30,30] clamp | KPP-4 |
| D-019 | human_detector | YOLO head channel validation | KPP-4 |
| D-020 | san_lte_redundancy | 2dB upgrade hysteresis (3GPP Qhyst) | Scenario D |
| D-021 | san_hub_slam | per-cell Bayesian voting | Gate-3 (4-UGV) |
| D-022 | san_rtk_gnss | $GxHDT parser + /heading Imu publisher | ★ KPP-1 |
| D-023 | san_rtk_gnss | fix-quality 별 yaw covariance | ★ KPP-1 |
| D-024 | san_role_management | /diagnostics/robot_status_audit (Hub-only) | Gate-3 진단 |
| D-025 | san_follower_tier | FollowerTarget QoS depth 20→50 | KPP-3 |
| D-026 | san_hub_slam | mismatch_ratio audit | 진단 |

### 7.2 DCN-2026-011 (Robot Profile Hardening, 4 sub-changes)

| DCN | 모듈 | 변경 사항 |
| --- | --- | --- |
| D-032 | san_operation_control + san_bringup | sbc_id parameter + priority resolver |
| D-033 | san_operation_control | inter-SBC heartbeat + RobotStatus sbc fields |
| D-034 | san_bringup | 5 robot profile launch wrappers |
| D-035 | infra/systemd | 5 systemd units + install.sh + Korean README |

### 7.3 Tier 2 (Phase-7 권고)

| PR | 모듈 | 변경 사항 |
| --- | --- | --- |
| PR-7 | san_hub_comm | GStreamer quality jump > 1 level 시 ramp smoothing |
| PR-8 | san_hub_slam | vote counter 주기 clear parameter |
| PR-9 | 3 modules | In-class default initializer (defensive) |

## 8. 향후 작업 (Phase-7, W18)

| 작업 | 목적 | 우선순위 |
| --- | --- | --- |
| PR-7 (bitrate ramp) | Demo Day 직후 Phase-7 안정화 | P1 |
| PR-8 (vote clear) | 24h+ endurance test 대응 | P2 |
| PR-9 (in-class init) | defensive coding | P2 |
| Aban Android App spec 갱신 | UI 의 새 토픽 5개 표시 | P1 |
| MAC auto-detection (옵션 A2) | DCN-2026-011 후속 — manual 운영 경험 후 검토 | P3 |
| Image build strategy 재검토 | Phase-7 endurance 결과 기반 | P3 |

## 9. SW 검증 — Gate 별 항목

### Gate-1 (6/5) — Deputy UGV 단독

- ☐ systemd unit 부팅 정상 (skyautonet-deputy.service)

- ☐ D-015/016/017 효과 확인 (5km/h 주행, 경계 cell phantom LETHAL 없음)

- ☐ D-022 RTK heading publish (dual-antenna 보유 시)

- ☐ RobotStatus 5Hz publish 정상

### Gate-2 (6/19) — Deputy + Follower 2-UGV

- ☐ Wi-Fi mesh 통신 ≤ 200ms latency

- ☐ Follower 가 FollowerTargetMessage (depth=50, D-025) 수신

- ☐ D-022 EKF imu1 wiring → /odometry/filtered/global heading 안정

### Gate-3 (7/3) — 4 UGVs (Hub + Deputy + Follower × 2)

- ☐ ★ Hub-only gate verify (P0 hotfix):

  - Follower 에서 ros2 topic info /diagnostics/robot_status_audit → Publisher count: 0

  - Hub 에서 → Publisher count: 1

- ☐ D-021 Bayesian voting — mismatch_ratio < 5% (4-robot SLAM 의견 일치)

- ☐ D-026 audit publisher 정상 작동

- ☐ Hub-Deputy takeover ≤ 10초 (Scenario D 사전 검증)

- ☐ Dual-SBC heartbeat (D-033): SBC #1 + SBC #2 mutual visibility

### Gate-4 (8/14) — Leader Go2 통합 + B/C/D

- ☐ Leader Go2 boot ≤ 90 sec

- ☐ Leader ↔ UGV DDS 통신

- ☐ All 4 scenarios 시연 완료

### Gate-5/6 (8/27, 9/2) — Dry-run + Code freeze

- ☐ 모든 P0/P1 defects 해결

- ☐ Release tag v1.5.5-demo

### Demo Day (9/4)

- ☐ 4-robot 통신 안정 (LTE redundancy verify)

- ☐ 15 min × 4 scenarios 절차 준수

- ☐ Q&A

## 10. 부록 — 패치 추적 매트릭스 (개발자용)

본 문서의 SW 동작은 다음 패치들 통합 가정:

Tier 1 (Gate-1 전 적용):
  PR-1_D-015-016-017_deputy_critical.patch    (45 KB)
  PR-2_D-018-019_human_detector.patch         (15 KB)
  PR-3_D-020_lte_hysteresis.patch             (15 KB)
  PR-4_D-021-026_hub_slam.patch               (26 KB)
  PR-5_D-022-023_rtk_kpp1.patch               (22 KB) ★ KPP-1
  PR-6_D-024_audit_hub_only_hotfix.patch      (13 KB) ★ P0 hotfix integrated

DCN-2026-011 (W5 적용):
  DCN-011-PR-1_D-032_sbc_id_parameter.patch
  DCN-011-PR-2_D-033_robot_status_sbc.patch
  DCN-011-PR-3_D-034_launch_wrappers.patch
  DCN-011-PR-4_D-035_systemd_units.patch

Tier 2 (Phase-7 권고):
  PR-7_bitrate_ramp_smoothing.patch
  PR-8_vote_counter_clear.patch
  PR-9_defensive_init.patch

모든 patches 는 SkyHunter_v1.5.2_FINAL_complete_bundle.zip + SkyHunter_Tier2_DCN-011_bundle.zip 에 포함.

Document Owner: 김태근 (PM, ㈜스카이오토넷) taegeun.kim@skyautonet.com / 010-3116-1310 Date: 2026-05-14 v1.0 발효