Interface Definition Specification

Operator Tablet <-> Swarm — 35 Standard Messages

Interface Definition Specification (IDS)

문서번호 SAN-IDS-CMD-001

v1.5

㈜스카이오토넷 / 군집제어형 소형 전술로봇 사업단

| Document ID | SAN-IDS-CMD-001 |
| --- | --- |
| Version | v1.5 |
| Classification | Interface Definition Specification |
| Preceding Docs | SAN-SDD-SWARM-001 v1.1 |
| Message Format | Google Protobuf v3 + ROS 2 .msg |
| Transport | Wi-Fi 6 Mesh (OpenWrt) + Military LTE + LoRa |
| Message Count | 39 (v1.0: 31 + 4 new in v1.1 + 2 new in v1.3 + 2 new in v1.5) |

# Revision History

| Version | Changes |
| --- | --- |
| v1.0 | Initial release. 31 standard messages (8 commands + 9 telemetry + 9 inter-robot + 5 infra). |
| v1.1 | All 31 v1.0 messages preserved with the following changes: (a) Added VideoStreamRequest in §3 (Android pull-mode video request); (b) Modified VideoStreamHandle in §4 to specify GStreamer SRT/UDP; (c) Added SurveillanceSectorAssignment, PanTiltCommand, SLAMAggregatedMap in §5 (3 new); (d) Replaced SLAMDelta with SLAMLocalDelta in §5 (collection period 1Hz -> 5s (v1.3 shortened)). Total 35 messages. |
| v1.4 | Deputy UGV message fields + new messages: (1) LeaderRoleAnnouncement message new (Section 5.15); (2) HubRoleAnnouncement message new (Section 5.16); (3) RobotStatus gains is_deputy_ugv, battery_percent fields; (4) deputy_chain default updated — [3, 2, ...] (Deputy 1st). |
| v1.5 | DCN-2026-001 applied (2026-05-12): (1) §5.15 LeaderRoleAnnouncement body formally added (was Revision-History-only); (2) §5.16 HubRoleAnnouncement body formally added; (3) §5.14 LTERoleAnnouncement lte_backup_chain default extended [3, 5] → [3, 4, 5, 6, 7, 8] (all UGVs now ship with LTE); (4) §1.2 Total 35 → 39 (catalog correction: v1.3 §5.13/§5.14 + v1.5 §5.15/§5.16); (5) §8.3 Inter-Robot Messages catalog 12 → 16 rows; (6) Related Docs version pins updated from v1.1 to v1.5. |

# Table of Contents

# 1. Overview

## 1.1 Document Scope

This document defines every communication message of the SAN swarm system in a single specification. Scope includes:

- Operator Android app -> Leader (command issuance)

- Leader -> Operator Android app (state/telemetry)

- Leader -> Follower (formation, predicted poses, surveillance sectors)

- Follower -> Leader/Hub (state, SLAM aggregated map)

- Hub UGV <-> wide-area C2 (military LTE backhaul)

## 1.2 Message Categories

| Category | Count | Direction |
| --- | --- | --- |
| §3 Operator Commands | 9 (+1 new) | Tablet -> Leader |
| §4 Swarm Telemetry | 9 (1 mod) | Swarm -> Tablet |
| §5 Inter-Robot | 16 (+3 new in v1.1, +1 mod, +2 new in v1.3, +2 new in v1.5) | Leader <-> Follower <-> Hub |
| §6 Infrastructure/System | 5 | Bidirectional |
| Total | 35 | — |

# 2. Message Architecture

## 2.1 Topic Naming Convention

/{robot_namespace}/{category}/{message_type}
    Examples:
    /leader/cmd/formation                  # operator -> leader
    /leader/cmd/surveillance_sectors        # operator -> leader (sector config)
    /swarm/predict/follower_target          # leader broadcast
    /swarm/surveillance/sector_assign       # leader -> followers (10s period)
    /hub/slam/aggregated_map                # Hub broadcast (5s (v1.3 shortened))
    /{robot_id}/cmd/pantilt                  # leader or operator -> robot
    /tablet/cmd/video_request                # tablet -> hub

## 2.2 Namespace Policy

| Namespace | Purpose | Example Topic |
| --- | --- | --- |
| /leader | Leader (Go2) exclusive | /leader/status, /leader/cmd |
| /hub | Hub UGV exclusive | /hub/slam, /hub/video_forward |
| /follower{N} | Each follower (N=1..7) | /follower1/status |
| /swarm | All-broadcast | /swarm/predict, /swarm/surveillance |
| /tablet | Operator app | /tablet/heartbeat |

## 2.3 QoS 4-Tier

| Priority | Reliability | Durability | History | Deadline | Example |
| --- | --- | --- | --- | --- | --- |
| P0 Critical | RELIABLE | TRANSIENT_LOCAL | KEEP_LAST(10) | 100 ms | FormationCommand, EmergencyStop |
| P1 Tactical | BEST_EFFORT/REL | VOLATILE | KEEP_LAST(3) | 200 ms | RobotStatus, FollowerTarget |
| P2 Operational | RELIABLE | TRANSIENT_LOCAL | KEEP_LAST(5) | 1 s | SLAMLocalDelta, SLAMAggregatedMap |
| P3 Background | BEST_EFFORT | VOLATILE | KEEP_LAST(1) | 5 s | LogEvent |

# 3. Operator Command Messages (Tablet -> Leader/Hub)

## 3.1 FormationCommand

| Attribute | Value |
| --- | --- |
| Direction | Tablet -> Leader |
| Topic | /leader/cmd/formation |
| QoS | P0, RELIABLE, KEEP_LAST(10) |
| Rate | Event |

message FormationCommand {
        enum FormationType {
            COLUMN=0; LINE=1; V_SHAPE=2; DIAMOND=3;
            ECHELON_LEFT=4; ECHELON_RIGHT=5; BOX=6;
            VEE_INVERTED=7; FREE_SPREAD=8;
        }
        FormationType  type             = 1;
        float          d                = 2;   // 1.5 <= d <= 20
        float          theta_deg        = 3;
        float          alpha_deg        = 4;
        float          transition_time  = 5;
        bool           dev_mode         = 6;
        uint64         timestamp_ms     = 7;
        string         requested_by     = 8;
    }

## 3.2 MissionStateCommand

message MissionStateCommand {
        enum State {
            IDLE=0; ASSEMBLY=1; OFFENSIVE=2; DEFENSIVE=3;
            PATROL=4; RTH=5; HOLD=6; EMERGENCY=7;
        }
        State    target_state     = 1;
        Pose2D   target_position  = 2;
        uint64   timestamp_ms     = 3;
        string   requested_by     = 4;
    }
    // QoS: P0, RELIABLE

## 3.3 FireAuthorization

message FireAuthorization {
        string    target_id           = 1;
        uint32    assigned_robot       = 2;
        enum FireMode {
            SINGLE_SHOT=0; THREE_BURST=1; FULL_AUTO=2; JAMMING=3;
        }
        FireMode  mode                = 3;
        Pose2D    target_pose         = 4;
        float     lead_distance_m     = 5;
        uint64    expires_at_ms       = 6;
        bytes     operator_signature  = 7;
        uint64    timestamp_ms        = 8;
    }
    // QoS: P0, RELIABLE, KEEP_ALL

## 3.4 JammingCommand

message JammingCommand {
        enum Band {
            BAND_2_4_GHZ=0; BAND_5_8_GHZ=1;
            BAND_GNSS_L1=2; BAND_GNSS_L2=3; BAND_ALL=4;
        }
        repeated Band  bands             = 1;
        Pose2D         target_direction  = 2;
        float          duration_s        = 3;
        float          power_w           = 4;
        uint64         timestamp_ms      = 5;
    }
    // QoS: P0, RELIABLE

## 3.5 WaypointCommand

message WaypointCommand {
        repeated Pose2D  waypoints         = 1;
        float            cruise_speed_mps  = 2;   // 0=auto, max 1.5
        bool             allow_replan      = 3;
        uint32           priority          = 4;
        uint64           timestamp_ms      = 5;
    }
    // QoS: P1, RELIABLE

## 3.6 ManualOverrideCommand

message ManualOverrideCommand {
        string  robot_id        = 1;
        enum Mode {
            DISENGAGE_AUTO=0; DIRECT_DRIVE=1; HOLD_POSITION=2; FOLLOW_ME=3;
        }
        Mode    mode             = 2;
        Velocity2D  manual_vel   = 3;
        uint64  expires_at_ms    = 4;
        uint64  timestamp_ms     = 5;
    }
    // QoS: P0, RELIABLE

## 3.7 EmergencyStop

message EmergencyStop {
        enum Severity {
            SOFT_STOP=0; HARD_STOP=1; DISARM=2; SHUTDOWN=3;
        }
        Severity  severity         = 1;
        string    reason           = 2;
        bytes     auth_signature   = 3;
        uint64    timestamp_ms     = 4;
    }
    // QoS: P0, RELIABLE, KEEP_ALL, LoRa fallback ON

## 3.8 OperatorHeartbeat

message OperatorHeartbeat {
        string    operator_id      = 1;
        bytes     auth_token       = 2;
        Pose2D    operator_pose    = 3;
        uint32    app_version      = 4;
        uint64    timestamp_ms     = 5;
    }
    // QoS: P1, BEST_EFFORT, 1 Hz

## 3.9 VideoStreamRequest (v1.1 new)

Pull-mode request issued by the Android app. Prevents unnecessary bandwidth and power consumption.

| Attribute | Value |
| --- | --- |
| Direction | Tablet -> Hub UGV (SBC #2) |
| Topic | /tablet/cmd/video_request |
| QoS | P1, RELIABLE |
| Rate | Event (operator taps View Video) |

message VideoStreamRequest {
        uint32  sequence            = 1;
        uint32  target_robot_id     = 2;
        string  protocol            = 3;   // "srt" | "udp"
        uint32  desired_port        = 4;
        string  codec               = 5;   // "h265" | "h264"
        string  quality             = 6;   // "hd" | "fhd" | "low" | "thumbnail"
        bool    encryption          = 7;
        string  passphrase_hint     = 8;
        string  action              = 9;   // "start" | "stop" | "change_quality"
        uint64  timestamp_ms        = 10;
    }

# 4. Swarm Telemetry Messages (Swarm -> Tablet)

## 4.1 RobotStatus

message RobotStatus {
        string  robot_id           = 1;
        enum Role { LEADER=0; HUB=1; FOLLOWER=2; }
        Role    role               = 2;
        Pose2D  pose               = 3;
        Velocity2D  velocity       = 4;
        float   heading_rad        = 5;
        enum Tier {
            T0_PREDICTIVE=0; T1_5_AUTO_REROUTE=1; T1_NORMAL=2;
            T2_CATCH_UP=3; T3_HARD_CATCH_UP=4; T4_BREADCRUMB=5;
        }
        Tier    current_tier       = 6;
        float   battery_percent    = 7;
        float   battery_voltage    = 8;
        float   cpu_load_percent   = 9;
        float   mem_used_percent   = 10;
        bool    rtk_fixed          = 11;
        bool    comm_healthy       = 12;
        bool    lte_active         = 13;   // v1.3: this robot is the active LTE gateway
        bool    is_lte_backup_designated = 14;  // v1.3: designated as LTE backup follower
        uint32  errors_active      = 13;
        uint64  timestamp_ms       = 14;
    }
    // QoS: P1, BEST_EFFORT, 1 Hz

## 4.2 FormationStatus

message FormationStatus {
        FormationType  active_type        = 1;
        float          d                  = 2;
        float          theta_deg          = 3;
        MissionState   mission_state      = 4;
        uint32         total_robots       = 5;
        uint32         healthy_robots     = 6;
        repeated string  unhealthy_ids    = 7;
        float          formation_error_m  = 8;
        bool           auto_column_active = 9;
        uint64         timestamp_ms       = 10;
    }
    // QoS: P1, BEST_EFFORT, 2 Hz

## 4.3 DetectedObject

message DetectedObject {
        string  object_id          = 1;
        string  detector_robot_id  = 2;
        enum ObjectClass {
            PERSON=0; VEHICLE=1; DRONE_FPV=2; DRONE_MULTI=3;
            DRONE_FIXED=4; UNKNOWN=5;
        }
        ObjectClass  obj_class    = 3;
        float        confidence   = 4;
        Pose2D       position     = 5;
        Velocity2D   velocity_estimate = 6;
        float        range_m      = 7;
        float        bearing_deg  = 8;
        bool         is_threat    = 9;
        uint32       threat_score = 10;
        bytes        thumbnail_jpeg = 11;
        uint64       detected_at_ms = 12;
    }
    // QoS: P0, RELIABLE, event

## 4.4 ThreatAlert

message ThreatAlert {
        string  alert_id          = 1;
        enum Severity {
            ADVISORY=0; CAUTION=1; WARNING=2; CRITICAL=3;
        }
        Severity  severity         = 2;
        enum ThreatType {
            DRONE_APPROACHING=0; ENEMY_PERSONNEL=1; ARTILLERY_DETECTED=2;
            AMBUSH_INDICATOR=3; COMM_JAMMING=4; GPS_JAMMING=5;
        }
        ThreatType  threat_type    = 3;
        string  related_object_id  = 4;
        float   time_to_impact_s   = 5;
        Pose2D  threat_position    = 6;
        string  message_text       = 7;
        uint64  timestamp_ms       = 8;
    }
    // QoS: P0, RELIABLE, KEEP_ALL

## 4.5 TierStatusChange

message TierStatusChange {
        string    robot_id          = 1;
        Tier      from_tier         = 2;
        Tier      to_tier           = 3;
        string    reason            = 4;
        float     delta_m           = 5;
        uint64    timestamp_ms      = 6;
    }
    // QoS: P0, RELIABLE, event

## 4.6 FireResult

message FireResult {
        string  fire_id              = 1;
        string  shooter_robot_id     = 2;
        string  target_object_id     = 3;
        uint32  rounds_fired         = 4;
        bool    target_neutralized   = 5;
        float   hit_probability      = 6;
        bytes   post_image_jpeg      = 7;
        uint64  fired_at_ms          = 8;
    }
    // QoS: P0, RELIABLE

## 4.7 BatteryWarning

message BatteryWarning {
        string  robot_id           = 1;
        enum Level { LOW=0; CRITICAL=1; SHUTDOWN=2; }
        Level   level              = 2;
        float   percent            = 3;
        uint32  minutes_remaining  = 4;
        bool    should_rth         = 5;
        uint64  timestamp_ms       = 6;
    }
    // QoS: P0, RELIABLE

## 4.8 VideoStreamHandle (v1.1 modified — GStreamer specified)

Hub UGV returns video stream handle to APP. v1.0's simple RTSP URL is now extended to SRT/UDP URI + encryption key + actual bitrate.

message VideoStreamHandle {
        uint32  sequence            = 1;
        uint32  target_robot_id     = 2;
        string  srt_uri             = 3;   // e.g. "srt://hub_ip:8888?mode=listener"
        string  codec               = 4;   // "h265" | "h264"
        string  quality             = 5;   // "hd" | "fhd" | "low" | "thumbnail"
        uint32  actual_bitrate_kbps = 6;
        string  passphrase          = 7;
        string  status              = 8;   // "streaming" | "stopped" | "error"
        string  error_msg           = 9;
        uint64  stream_start_ms     = 10;
        uint64  timestamp_ms        = 11;
    }
    // QoS: P1, RELIABLE

## 4.9 SLAMUpdate

message SLAMUpdate {
        Pose2D  origin              = 1;
        float   resolution_m        = 2;
        uint32  width_cells         = 3;
        uint32  height_cells         = 4;
        bytes   occupancy_delta     = 5;
        uint32  contributing_robots = 6;
        uint64  timestamp_ms        = 7;
    }
    // QoS: P2, RELIABLE, 0.5 Hz (for tablet screen refresh)

# 5. Inter-Robot Messages

## 5.1 FollowerTargetMessage (1s prediction)

| Attribute | Value |
| --- | --- |
| Direction | Leader -> Follower (broadcast) |
| Topic | /swarm/predict/follower_target |
| QoS | P0, RELIABLE, KEEP_LAST(3) |
| Rate | 10 Hz (100 ms) |
| Size | ~200 byte |

message FollowerTargetMessage {
        uint32  follower_id            = 1;
        Pose2D  target_pose            = 2;
        uint64  valid_at_ms            = 3;
        repeated Pose2D  leader_path_segment = 4;
        uint32  sequence_num           = 5;
        float   confidence             = 6;
        Velocity2D  target_velocity    = 7;
        uint64  generated_at_ms        = 8;
    }
    message Pose2D {
        double  x        = 1;
        double  y        = 2;
        float   heading  = 3;
    }
    message Velocity2D {
        float   vx       = 1;
        float   vy       = 2;
        float   omega    = 3;
    }

## 5.2 BreadcrumbBroadcast

message BreadcrumbBroadcast {
        uint64  timestamp_us       = 1;
        Pose2D  position           = 2;
        float   speed_mps          = 3;
        enum TerrainClass {
            OPEN=0; FOREST=1; URBAN=2; NARROW=3; CLIFF=4;
        }
        TerrainClass  terrain        = 4;
        uint8   formation_id       = 5;
        uint32  sequence_num       = 6;
    }
    // QoS: P1, RELIABLE, KEEP_LAST(50), 0.5-2 Hz (1m moved or 0.5s)

## 5.3 LeaderElection (Modified Raft)

message LeaderElection {
        enum MessageType {
            REQUEST_VOTE=0; VOTE_GRANTED=1; VOTE_DENIED=2;
            LEADER_ANNOUNCE=3; HEARTBEAT=4;
        }
        MessageType  type                = 1;
        uint32       term                = 2;
        string       candidate_id        = 3;
        float        candidate_priority  = 4;   // is_hub_ugv top priority
        string       voter_id            = 5;
        uint64       timestamp_ms        = 6;
    }
    // QoS: P0, RELIABLE, KEEP_ALL, event (Hub UGV is priority 1)

## 5.4 EmergencyAlert

message EmergencyAlert {
        string  source_robot_id  = 1;
        enum AlertType {
            OBSTACLE_AHEAD=0; COLLISION_WARNING=1; CLIFF_DETECTED=2;
            FORMATION_BREAK=3; SENSOR_FAIL=4;
        }
        AlertType  type             = 2;
        Pose2D     alert_position   = 3;
        float      severity         = 4;
        string     description      = 5;
        uint64     timestamp_ms     = 6;
    }
    // QoS: P0, RELIABLE, event

## 5.5 SLAMLocalDelta (v1.1 modified — 1Hz -> 5s (v1.3 shortened) aggregated)

Replaces the v1.0 SLAMDelta (1 Hz). Sends an accumulated aggregated map every 5s (v1.3 shortened), reducing comm load by 1/100.

message SLAMLocalDelta {
        uint32  sequence                  = 1;
        string  robot_id                  = 2;
        bytes   occupancy_grid_delta_png  = 3;   // PNG compressed
        Pose2D  origin                    = 4;
        float   resolution_m              = 5;
        uint64  coverage_start_ms         = 6;   // aggregation start time
        uint64  coverage_end_ms           = 7;   // aggregation end time
        uint64  timestamp_ms              = 8;
    }
    // QoS: P2, RELIABLE
    // Direction: all robots (Leader + 7 Follower)
    // Receiver: Hub UGV SBC #1
    // Rate: 5s (v1.3 shortened) (default 30s, dynamic adjustment)

## 5.6 SLAMAggregatedMap (v1.1 new)

Broadcast of the merged global map by Hub UGV SBC #1. Updates the Nav2 cost map static layer on every robot.

message SLAMAggregatedMap {
        uint32  sequence            = 1;
        bytes   occupancy_grid_png  = 2;   // PNG compressed global map
        Pose2D  origin              = 3;
        float   resolution_m        = 4;
        uint32  width_cells         = 5;
        uint32  height_cells        = 6;
        uint32  contributing_robots = 7;
        uint64  timestamp_ms        = 8;
    }
    // QoS: P2, RELIABLE, TRANSIENT_LOCAL
    // Sender: Hub UGV (SBC #1)
    // Receiver: all robots + operator tablet
    // Rate: 5s (v1.3 shortened) (dynamic)

## 5.7 GlobalMapBroadcast (v1.0 retained, operator screen)

message GlobalMapBroadcast {
        Pose2D  map_origin          = 1;
        float   resolution_m        = 2;
        uint32  width               = 3;
        uint32  height              = 4;
        bytes   full_map            = 5;
        bytes   delta_map           = 6;
        repeated string contributor_ids = 7;
        uint64  timestamp_ms        = 8;
    }
    // QoS: P2, RELIABLE, TRANSIENT_LOCAL, 0.5 Hz

## 5.8 HeartBeat

message HeartBeat {
        string  robot_id          = 1;
        uint32  sequence_num      = 2;
        uint64  uptime_ms         = 3;
        float   cpu_load_percent  = 4;
        bool    is_leader         = 5;
        uint64  timestamp_ms      = 6;
    }
    // QoS: P1, BEST_EFFORT, 1 Hz

## 5.9 SlotAssignment

message SlotAssignment {
        string  follower_id        = 1;
        Pose2D  offset_in_body     = 2;
        uint32  transition_id      = 3;
        float   transition_time_s  = 4;
        uint32  priority           = 5;
        uint64  valid_from_ms      = 6;
        uint64  timestamp_ms       = 7;
    }
    // QoS: P0, RELIABLE, event (on formation change, Hungarian result)

## 5.10 SurveillanceSectorAssignment (v1.1 new)

Leader assigns surveillance sectors to each follower. Published every 10s (default) or immediately on events.

| Attribute | Value |
| --- | --- |
| Direction | Leader -> all Followers + Hub UGV (broadcast) |
| Topic | /swarm/surveillance/sector_assign |
| QoS | P1, RELIABLE |
| Rate | 10s (periodic) + event (formation change, threat) |

message SurveillanceSectorAssignment {
        uint32  sequence              = 1;
        uint32  robot_id              = 2;          // target
        float   sector_start_deg      = 3;          // -180 to 180 (heading-based)
        float   sector_end_deg        = 4;
        uint32  valid_period_sec      = 5;          // default 10
        string  priority              = 6;          // "primary" | "threat_focus" | "overlap"
        string  mode_hint             = 7;          // "sweep" | "track" | "fixed"
        uint64  timestamp_ms          = 8;
    }

## 5.11 PanTiltCommand (v1.1 new)

Leader or operator controls a robot's pan-tilt. Followers may also self-publish to start sweep upon receiving sector assignment.

| Attribute | Value |
| --- | --- |
| Direction | Leader or Tablet -> Robot |
| Topic | /{robot_id}/cmd/pantilt |
| QoS | P1, RELIABLE |
| Rate | Event or on sweep entry |

message PanTiltCommand {
        uint32  sequence            = 1;
        uint32  robot_id            = 2;
        float   target_pan_deg      = 3;          // heading-based
        float   target_tilt_deg     = 4;
        float   speed_dps           = 5;           // degrees/sec
        string  mode                = 6;           // "sweep" | "track" | "fixed" | "engage"
        float   sweep_range_deg     = 7;           // sweep width
        uint64  timestamp_ms        = 8;
    }

## 5.12 SwarmHealthSummary

message SwarmHealthSummary {
        uint32  total_robots          = 1;
        uint32  online_robots         = 2;
        float   avg_battery_percent   = 3;
        float   min_battery_percent   = 4;
        float   avg_formation_err_m   = 5;
        float   comm_quality_percent  = 6;
        uint32  robots_in_t4          = 7;
        uint32  robots_in_emergency   = 8;
        uint64  timestamp_ms          = 9;
    }
    // QoS: P1, BEST_EFFORT, 1 Hz, Hub -> Tablet

## 5.13 CostMapUpdate (v1.3 new)

Each UGV publishes its computed Local Cost Map. Hub UGV SBC #1 collects them for cross-validation.

| Attribute | Value |
| --- | --- |
| Direction | Each UGV -> broadcast |
| Topic | /{robot_id}/local/cost_map |
| QoS | P2, BEST_EFFORT, KEEP_LAST(1) |
| Rate | 1 Hz (KPP: cost_map_latency_p99 <= 5s) |

## 5.14 LTERoleAnnouncement (v1.3 new)

LTE gateway promotion/demotion broadcast. When Hub UGV (S2) LTE fails, Follower S3 (1st priority) auto-promotes.

| Attribute | Value |
| --- | --- |
| Direction | LTE backup promoting/demoting robot -> broadcast |
| Topic | /swarm/lte/role_announce |
| QoS | P0, RELIABLE, KEEP_ALL |
| Rate | Event (on promotion/demotion/re-affirm) |

## 5.15 LeaderRoleAnnouncement (★ v1.5 new — body formally added per DCN-2026-001 D-005)

Driver of the 4-tier Leader succession chain defined in SAN-SDD-SWARM-001 v1.5 §5.6. Each candidate self-declares with a CANDIDATE announcement after a priority-tier grace period (200 ms × priority); the highest-priority candidate wins by emitting a PROMOTED. Split-brain prevention via monotonically increasing leader_term.

| Attribute | Value |
| --- | --- |
| Direction | Candidate / new leader → broadcast |
| Topic | /swarm/leader/role_announce |
| QoS | P0, RELIABLE, TRANSIENT_LOCAL, KEEP_LAST(10) |
| Rate | Event (on CANDIDATE / PROMOTED / DEMOTED transition) |
| Implementing package | san_role_management |
| Tested in | SAN-TST-INT-001 v1.5 S18-1, S18-2, S18-4 |

// SAN v1.5 IDS §5.15 — Leader 4-tier succession broadcast

message LeaderRoleAnnouncement {

enum Role {

LEADER_NORMAL    = 0;   // current leader operating normally, or non-leader robot

LEADER_CANDIDATE = 1;   // self-declared candidate, grace-period in progress

LEADER_PROMOTED  = 2;   // succession complete, this robot is current leader

LEADER_DEMOTED   = 3;   // original leader recovered, this robot demoted

}

enum SuccessionPriority {

PRIORITY_UNSPECIFIED  = 0;

PRIORITY_DEPUTY       = 1;   // 1st: Deputy UGV (S3)

PRIORITY_HUB          = 2;   // 2nd: Hub UGV (S2)

PRIORITY_BATTERY_MAX  = 3;   // 3rd: max-battery follower

PRIORITY_LIMP_MODE    = 4;   // 4th: no eligible candidate → Limp Mode

}

Header             header              = 1;

uint32             sequence            = 2;

uint32             robot_id            = 3;

uint32             leader_term         = 4;   // monotonic, split-brain prevention

Role               role                = 5;

SuccessionPriority succession_priority = 6;

float              battery_percent     = 7;   // 0.0 ~ 100.0 (priority 3 comparison)

string             reason              = 8;   // "leader_heartbeat_timeout" | "leader_recovered" | "battery_lowest"

uint64             timestamp_ms        = 9;

}

### 5.15.1 Split-brain Prevention Rule

Receivers compare incoming leader_term with their cached value: (a) incoming < cached → discard as stale, log WARN; (b) incoming > cached → adopt new term and respect role transition; (c) incoming = cached, different robot_id → lowest robot_id wins (tie-breaker).

### 5.15.2 Grace Period Timing

Leader heartbeat timeout = 1400 ms (200 ms × 7 missed samples). On timeout, candidates wait priority × 200 ms before self-promoting: DEPUTY (S3) 200 ms grace → promotes at t+1.6 s after Leader failure; HUB (S2) 400 ms grace → promotes at t+1.8 s only if no DEPUTY announcement seen; BATTERY_MAX 600 ms grace → promotes at t+2.0 s only if no DEPUTY/HUB announcement.

## 5.16 HubRoleAnnouncement (★ v1.5 new — body formally added per DCN-2026-001 D-005)

Hub UGV (S2) heartbeat 5-second timeout triggers Deputy UGV (S3) to broadcast HUB_PROMOTED while assuming LTE gateway + SLAM aggregation + GStreamer SRT relay duties (full takeover). On Hub recovery, the Deputy emits HUB_DEMOTED. If both Hub and Deputy are down, the LimpModeManager (see SAN-SDD-SWARM-001 v1.5 §5.7) detects the gap and broadcasts LIMP_MODE_ACTIVE on a separate channel — no HUB_PROMOTED is emitted in that case.

| Attribute | Value |
| --- | --- |
| Direction | Promoting / demoting Hub-role robot → broadcast |
| Topic | /swarm/hub/role_announce |
| QoS | P0, RELIABLE, TRANSIENT_LOCAL, KEEP_LAST(10) |
| Rate | Event (on FAILING / PROMOTED / DEMOTED transition) |
| Implementing package | san_role_management |
| Tested in | SAN-TST-INT-001 v1.5 S18-3, S18-5, S18-6 |

// SAN v1.5 IDS §5.16 — Hub-Deputy 이중화 broadcast

message HubRoleAnnouncement {

enum Role {

HUB_NORMAL   = 0;   // current Hub operating normally, or non-Hub robot

HUB_FAILING  = 1;   // Hub heartbeat delay detected (Deputy succession pending)

HUB_PROMOTED = 2;   // Deputy assumed Hub role (LTE + SLAM + Video all transferred)

HUB_DEMOTED  = 3;   // original Hub recovered, Deputy demoted

}

Header  header                  = 1;

uint32  sequence                = 2;

uint32  robot_id                = 3;

uint32  hub_term                = 4;   // monotonic, split-brain prevention

Role    role                    = 5;

bool    lte_active              = 6;   // LTE gateway active

bool    slam_aggregation_active = 7;   // SLAM aggregation active

bool    video_relay_active      = 8;   // GStreamer SRT relay active

string  reason                  = 9;   // "hub_heartbeat_timeout" | "hub_recovered" | "init"

uint64  timestamp_ms            = 10;

}

### 5.16.1 Full Takeover Semantics

"HUB_PROMOTED" implies all three capability flags are simultaneously true. Partial takeovers (only LTE, or only SLAM) are NOT valid Hub roles — they would be advertised via LTERoleAnnouncement (§5.14) only.

### 5.16.2 Recovery Sequence

When original Hub (S2) heartbeat resumes: (1) S2 broadcasts HubRoleAnnouncement(HUB_NORMAL, hub_term=M+1) with intent to reclaim; (2) Deputy (S3) compares S2 batteries / SBC health against own; if S2 nominal, S3 broadcasts HUB_DEMOTED with hub_term=M+1 and ceases LTE/SLAM/Video relay; (3) S2 resumes Hub duties; S3 returns to standby. Verified in TST S18-6 (≤ 5 s exit).

# 6. Infrastructure/System Messages

## 6.1 TopicCatalog

message TopicCatalog {
        string  robot_id           = 1;
        repeated TopicEndpoint  publishers   = 2;
        repeated TopicEndpoint  subscribers  = 3;
        uint64  generated_at_ms    = 4;
    }
    message TopicEndpoint {
        string  topic_name      = 1;
        string  message_type    = 2;
        string  qos_profile     = 3;
    }
    // QoS: P1, RELIABLE, TRANSIENT_LOCAL, event (startup/change)

## 6.2 VersionNegotiation

message VersionNegotiation {
        string         peer_id             = 1;
        uint32         schema_major        = 2;
        uint32         schema_minor        = 3;
        uint32         sw_version          = 4;
        repeated uint32 supported_majors   = 5;
        bool           is_compatible       = 6;
        string         error_message       = 7;
        uint64         timestamp_ms        = 8;
    }
    // QoS: P0, RELIABLE

## 6.3 SecurityHandshake

message SecurityHandshake {
        string  peer_id              = 1;
        bytes   certificate_chain    = 2;   // X.509 v3
        bytes   nonce                = 3;
        bytes   signature            = 4;   // RSA 4096
        string  cipher_suite         = 5;   // "AES-256-GCM"
        uint64  timestamp_ms         = 6;
    }
    // QoS: P0, RELIABLE

## 6.4 LogEvent

message LogEvent {
        string  source_id        = 1;
        enum Severity {
            DEBUG=0; INFO=1; WARN=2; ERROR=3; FATAL=4;
        }
        Severity  severity         = 2;
        string  component        = 3;
        string  message          = 4;
        string  correlation_id   = 5;
        uint64  timestamp_ms     = 6;
    }
    // QoS: P3, BEST_EFFORT

## 6.5 TimeSync

message TimeSync {
        uint64  master_time_us     = 1;
        int64   offset_us          = 2;
        float   drift_ppm          = 3;
        uint32  sync_round         = 4;
    }
    // QoS: P0, BEST_EFFORT, 1 Hz, Hub master -> all robots
    // PTP IEEE-1588 grandmaster on Hub UGV SBC #1

# 7. Security (DDS-Security)

| Function | Standard | Adopted Value |
| --- | --- | --- |
| Authentication | OMG DDS-Security AuthN | PKI X.509 v3, RSA 4096 |
| Encryption | OMG DDS-Security Crypto | AES-256-GCM |
| Access Control | OMG DDS-Security AccessCtrl | Permissions Document |
| Logging | OMG DDS-Security Logging | Centralized on Hub UGV |
| Key Management | Internal policy | Hub UGV TPM, per-mission rotation |

## 7.1 Video Stream Security

- SRT built-in AES-128 encryption (per-mission passphrase)

- Hub-Operator segment: IPsec or OpenVPN tunnel (additional)

- Robot-Hub LAN: WPA3-SAE + DDS Security

## 7.2 Emergency Key Revocation

On suspected robot loss/capture, the operator app's "Compromise" button immediately revokes that robot's certificate. The CRL is broadcast to the entire swarm immediately.

# 8. Appendix — Message Catalog (35 total)

## 8.1 Command Messages (9)

| # | Name | Topic | QoS | Rate | v1.1 |
| --- | --- | --- | --- | --- | --- |
| 3.1 | FormationCommand | /leader/cmd/formation | P0 | evt |  |
| 3.2 | MissionStateCommand | /leader/cmd/mission_state | P0 | evt |  |
| 3.3 | FireAuthorization | /leader/cmd/fire_auth | P0 | evt |  |
| 3.4 | JammingCommand | /leader/cmd/jamming | P0 | evt |  |
| 3.5 | WaypointCommand | /leader/cmd/waypoint | P1 | evt |  |
| 3.6 | ManualOverrideCommand | /{robot}/cmd/manual_override | P0 | evt |  |
| 3.7 | EmergencyStop | /swarm/emergency_stop | P0 | evt |  |
| 3.8 | OperatorHeartbeat | /leader/heartbeat/operator | P1 | 1 Hz |  |
| 3.9 | VideoStreamRequest | /tablet/cmd/video_request | P1 | evt | * new |

## 8.2 Telemetry Messages (9)

| # | Name | Topic | QoS | Rate | v1.1 |
| --- | --- | --- | --- | --- | --- |
| 4.1 | RobotStatus | /{robot}/status/robot | P1 | 1 Hz |  |
| 4.2 | FormationStatus | /leader/status/formation | P1 | 2 Hz |  |
| 4.3 | DetectedObject | /{robot}/detected/object | P0 | evt |  |
| 4.4 | ThreatAlert | /leader/alert/threat | P0 | evt |  |
| 4.5 | TierStatusChange | /{robot}/status/tier_change | P0 | evt |  |
| 4.6 | FireResult | /{robot}/status/fire_result | P0 | evt |  |
| 4.7 | BatteryWarning | /{robot}/warn/battery | P0 | evt |  |
| 4.8 | VideoStreamHandle | /leader/status/video_handle | P1 | evt | * mod |
| 4.9 | SLAMUpdate | /hub/slam/global_map_delta | P2 | 0.5 Hz |  |

## 8.3 Inter-Robot Messages (16)

| # | Name | Topic | QoS | Rate | v1.1 |
| --- | --- | --- | --- | --- | --- |
| 5.1 | FollowerTargetMessage | /swarm/predict/follower_target | P0 | 10 Hz |  |
| 5.2 | BreadcrumbBroadcast | /swarm/breadcrumb | P1 | 0.5-2 Hz |  |
| 5.3 | LeaderElection | /swarm/leader_election | P0 | evt |  |
| 5.4 | EmergencyAlert | /swarm/alert/emergency | P0 | evt |  |
| 5.5 | SLAMLocalDelta | /{robot}/slam/local_delta | P2 | 5s (v1.3 shortened) | * mod |
| 5.6 | SLAMAggregatedMap | /hub/slam/aggregated_map | P2 | 5s (v1.3 shortened) | * new |
| 5.7 | GlobalMapBroadcast | /swarm/slam/global_map | P2 | 0.5 Hz |  |
| 5.8 | HeartBeat | /{robot}/heartbeat | P1 | 1 Hz |  |
| 5.9 | SlotAssignment | /{follower}/cmd/slot_assignment | P0 | evt |  |
| 5.10 | SurveillanceSectorAssignment | /swarm/surveillance/sector_assign | P1 | 10 s | * new |
| 5.11 | PanTiltCommand | /{robot}/cmd/pantilt | P1 | evt | * new |
| 5.12 | SwarmHealthSummary | /hub/health/swarm_summary | P1 | 1 Hz |  |

## 8.4 Infrastructure/System Messages (5)

| # | Name | Topic | QoS | Rate | v1.1 |
| --- | --- | --- | --- | --- | --- |
| 6.1 | TopicCatalog | /{robot}/system/topic_catalog | P1 | evt |  |
| 6.2 | VersionNegotiation | /swarm/system/version_neg | P0 | evt |  |
| 6.3 | SecurityHandshake | /swarm/system/security_handshake | P0 | evt |  |
| 6.4 | LogEvent | /{robot}/system/log | P3 | evt |  |
| 6.5 | TimeSync | /swarm/system/time_sync | P0 | 1 Hz |  |

## 8.5 v1.0 -> v1.1 Change Summary

| Change | Messages | Rationale |
| --- | --- | --- |
| New (+4) | VideoStreamRequest, SurveillanceSectorAssignment, PanTiltCommand, SLAMAggregatedMap | v1.1 four design changes |
| Modified (2) | VideoStreamHandle (GStreamer SRT specified), SLAMLocalDelta (replaces SLAMDelta, 5s (v1.3 shortened)) | Protocol specification + period adjustment |
| Retained (29) | All other v1.0 messages | — |
| Total | 35 messages | — |