Surveillance / SLAM / Video Streaming

Supplementary Design Document

Sub-document to Unified SDD v1.3

문서번호 SAN-SDD-SUR-001

v1.1

㈜스카이오토넷 / 군집제어형 소형 전술로봇 사업단

| Document ID | SAN-SDD-SUR-001 |
| --- | --- |
| Version | v1.4 |
| Classification | Supplementary SDD (sub to Unified SDD) |
| Preceding Docs | SAN-SDD-SWARM-001 v1.4, SAN-IDS-CMD-001 v1.1 |

# Revision History

| Version | Changes |
| --- | --- |
| v1.0 | Initial release. 4 design changes specified. |
| v1.1 | Cover classification table updated (author/schedule removed). |
| v1.2 | PM decisions D1~D7 + A1~A5 applied: (D1) Squadron Follower loss (8 -> 7) (Leader S1 + Hub UGV S2 + Followers S3-S8 = 6 followers); (D2) Leader succession deputy_chain = [2, 3, 4, 5, 6, 7, 8], Hub UGV(S2) is priority 1; (D3) CycloneDDS NIC binding specified; (D7) AI inference Phase 1 RK3588 / PoC Hailo M.2; Section 3 sector distribution table updated for 8 robots, Section 4 Hub UGV S2 specified. |
| v1.3 | Cost Map + SLAM 5s + LTE Redundancy: (1) §5 SLAM aggregation period **5s** (v1.3) -> **5 sec**; (2) §4 Robosense E1 VFOV **25° -> 90°** correction; (3) UGV spec table (1300x850mm, 12km/h, slope 30°, obstacle 235mm, ditch 220mm); (4) LTE redundancy — Follower S3 LTE backup activates on Hub LTE failure. |
| v1.4 | Deputy UGV pre-designation reflected: (1) Section 5 SLAM aggregation — Deputy UGV (S3) takes over on Hub UGV failure; (2) Deputy UGV spec is identical to Hub UGV (dual SBC + LTE). |

# Table of Contents

# 1. Overview

## 1.1 Document Position

This Supplementary SDD is a sub-document of SAN-SDD-SWARM-001 v1.4 (Unified SDD), specifying the following four design decisions formally:

- 8-robot 360° surveillance distribution — Leader assigns each follower a front-180° area and Hub UGV takes the rear 180°

- Follower Pan-tilt low-speed sweep during driving — active 360° coverage while moving

- Hub UGV dual SBC architecture — split between SLAM fusion SBC and Comm/Video SBC

- SLAM collection period change (1 Hz → **5s** (v1.3)) and video streaming GStreamer UDP/SRT adoption

## 1.2 Change Summary

| # | Topic | Previous (v1.0) | This Document |
| --- | --- | --- | --- |
| S1 | Surveillance distribution | Not specified (forward-only recon) | Dynamic Leader-driven 360° share (10s) |
| S2 | Follower Pan-tilt | Only when tracking target | Slow sweep while driving (30°/sec) |
| S3 | Hub UGV SBC | Single RK3588J | Dual SBC (SLAM + Comm/Video split) |
| S4 | SLAM collection period | Follower -> Hub 1 Hz | Aggregated bundle every **5s** (v1.3) |
| S5 | Video streaming | Protocol unspecified | GStreamer UDP/SRT, Android pull mode |

## 1.3 KPP Impact

- **Real-time video ≤ 200ms** — GStreamer SRT live latency=120ms tune achieves target

- **Target classification ≥ 90%** — Active Pan-tilt sweep minimizes blind spots

- **Swarm comm latency ≤ 150ms** — Relaxed SLAM period frees bandwidth headroom

# 2. Camera Surveillance Area Definition

## 2.1 Camera HFOV Specifications

| Mode | Use | Zoom | HFOV (Day) | HFOV (Thermal) |
| --- | --- | --- | --- | --- |
| Surveillance | Wide sweep | 1x (Wide) | ~60° | ~24° |
| Observation | POI track | 3-5x | ~12-20° | ~5-8° |
| Identification | Target ID | 10-15x | ~4-6° | ~1.5-2.5° |
| Engagement | Precision aim | 20-30x | ~2-3° | ~0.8-1.2° |

Unless otherwise specified, Surveillance Mode (Wide, HFOV 60°) is the default. For wide-area surveillance, 60° HFOV provides ≥ 30° overlap between adjacent followers.

## 2.2 Leader (Go2) Fixed Surveillance

- Direction: Forward 60° (heading-aligned)

- Range: ±30° (30° each side of heading)

- Zoom: 1x Wide (HFOV 60°) fixed

- Data: Real-time video to Hub UGV (optional broadcast)

The Leader's surveillance area auto-tracks heading. Leader focuses on 1s prediction computation and recon, so low camera autonomy simplifies system complexity.

## 2.3 Follower Pan-tilt Dynamic Surveillance

### 2.3.1 Pan-tilt Operation Modes

| Mode | Pan Motion | Tilt Motion | Transition Trigger |
| --- | --- | --- | --- |
| Sweep | Reciprocating in sector | Horizon-based -5° to +10° | Default while driving |
| Fixed | Hold commanded direction | Hold commanded angle | Leader command or operator |
| Track | Auto-follow target | Auto-follow target | AI detection or threat |
| Engage | Target lock-on + lead | Target lock-on + lead | Just before Confirm Fire |

### 2.3.2 Sweep Rotation Speed

ω_sweep = (sector_width - HFOV) × 2 / period
    Example: sector_width = 90°, HFOV = 60°, period = 6s
             ω_sweep = (90 - 60) × 2 / 6 = 10°/sec
    Example: sector_width = 180°, HFOV = 60°, period = 8s
             ω_sweep = (180 - 60) × 2 / 8 = 30°/sec

Standard sweep speed 30°/sec. On threat detection, immediate switch to Track mode for target lock-on. Night thermal mode uses half speed (15°/sec) due to narrower HFOV (~24°).

### 2.3.3 Pan-tilt Stabilization During Driving

Vehicle vibration on pan-tilt video is absorbed by gimbal stabilization. The RCWS gimbal includes active IMU-based stabilization meeting the 0.05° tracking error KPP. Sweep mode is area scanning (not target tracking), tolerating wider vibration than Track mode.

## 2.4 Hub UGV Surveillance Area

- Pan range: heading +90° to +270° (rear 180°)

- Sweep speed: 30°/sec (8s period)

- Threat detected: immediate Track mode + position broadcast to nearby followers

# 3. Surveillance Area Distribution Policy

## 3.1 9-Robot 360° Distribution Principles

| Robot | ID | Direction | Primary Mode | Role |
| --- | --- | --- | --- | --- |
| Leader (Go2) | S1 | Front 0° ±30° | Fixed (front) | Recon + path clearance |
| Hub UGV | S2 | Rear ±90° (180°) | Sweep 30°/sec | Primary rear surveillance + comm/SLAM hub + Leader succession priority 1 |
| F1 (Follower) | S3 | Front-left -80° to -30° | Sweep | Front-left surveillance |
| F2 (Follower) | S4 | Front-right +30° to +80° | Sweep | Front-right surveillance |
| F3 (Follower) | S5 | Left flank -130° to -80° | Sweep | Left flank |
| F4 (Follower) | S6 | Right flank +80° to +130° | Sweep | Right flank |
| F5 (Follower) | S7 | Left rear -180° to -130° | Sweep | Left rear flank |
| F6 (Follower) | S8 | Right rear +130° to +180° | Sweep | Right rear flank |

## 3.2 Front 180° Distribution Algorithm

| Robot | ID | Sector | Center | Primary Direction |
| --- | --- | --- | --- | --- |
| Leader | S1 | -30° to +30° | 0° (front) | Front 60° |
| Hub UGV | S2 | +90° to -90° (wrap) | 180° (rear) | Rear 180° sweep |
| F1 | S3 | -80° to -30° | -55° (front-left) | Front-left |
| F2 | S4 | +30° to +80° | +55° (front-right) | Front-right |
| F3 | S5 | -130° to -80° | -105° (left flank) | Left flank |
| F4 | S6 | +80° to +130° | +105° (right flank) | Right flank |
| F5 | S7 | -180° to -130° | -155° (left rear) | Left rear flank |
| F6 | S8 | +130° to +180° | +155° (right rear) | Right rear flank |

Each follower's assigned sector is 60° wide and camera HFOV is 60°, so sweep can degenerate to fixed gaze. For active gimbal operation, sectors are usually allocated slightly wider (80-90°) with 30°/sec sweep.

## 3.3 Hub UGV Rear 180° Coverage

- Pan range: chassis heading +90° to -90° (rear hemisphere)

- Sweep speed: 30°/sec

- Sweep period: ~8s (180° + 60° HFOV round trip)

- Threat detected: Track mode + cross-cue with F6, F7

Rear surveillance counters ambush and flanking infiltration. Combination of Hub's central-rear position and 360° pan-tilt capability ensures rear security.

## 3.4 Periodic Refresh

| Trigger | Refresh Period | Refresh Scope |
| --- | --- | --- |
| Periodic refresh | 10s | Recompute all follower sectors |
| Formation change | Immediate (post-transition) | Full reassign for new formation |
| Follower lost | Immediate (post-reform) | Redistribute 360° among remaining |
| Threat detected | Immediate (while present) | Priority assign to threat-side followers |
| Manual command | Operator immediate | Concentrate/disperse over specific area |

10s period rationale: (1) Longer than sweep period (~8s), avoiding mid-sweep sector changes, (2) Fast enough for formation drift, (3) Negligible comm load (35 B/s for 7 followers). Adjustable 5-30s by PM.

## 3.5 Dynamic Adaptation

### 3.5.1 Threat Sector Focus

When RF scanner or any follower detects a threat, Leader reassigns 2-3 closest followers to the same threat sector. Yields (1) higher detection probability, (2) better cross-cueing accuracy, (3) faster engagement.

### 3.5.2 Gap Filling

On follower loss (comm loss, power loss), remaining followers redistribute sectors automatically. 9 → 8 transition increases average sector width 30° → 35.7°, full coverage guaranteed.

### 3.5.3 Recon vs Defense Mode

Offensive mode (assault): narrow front sectors (15-20°/follower), wide rear. Defense mode (point defense): balanced (45°/follower) front-rear.

# 4. Hub UGV Dual SBC Architecture

## 4.1 Dual SBC Rationale

Hub UGV (robot_id = S2) used a single RK3588J SBC in v1.0. The following v1.1 concurrent requirements clearly exceed single-SBC capacity:

- 9-robot SLAM data fusion (multirobot_map_merge + g2o)

- Wi-Fi 6 Mesh routing oversight (OpenWrt batman-adv)

- Military LTE gateway (mwan3)

- Video stream forwarding (GStreamer SRT/UDP)

- DDS Security encrypt/decrypt (AES-256-GCM)

- 360° surveillance dispatch

→ RK3588J would exceed 90% utilization causing latency spike, packet drop, SLAM delays. Solution: deploy two SBCs to split workload.

## 4.2 Dual SBC Role Split

| Aspect | SBC #1 (SLAM) | SBC #2 (Comm/Video) |
| --- | --- | --- |
| Model | RK3588J or Jetson Orin | RK3588J |
| Memory | 16 GB LPDDR5 | 8 GB LPDDR5 |
| Storage | 256 GB NVMe | 128 GB NVMe |
| Primary task | SLAM aggregated fusion | Wi-Fi6 Mesh + LTE gateway |
| Secondary task | Global map broadcast | GStreamer SRT/UDP forwarding |
| Processes | slam_toolbox, multirobot_map_merge | DDS, GStreamer, OpenVPN/IPsec |
| Network | Internal GbE (direct to SBC#2) | Wi-Fi 6 (ext) + LTE modem + Internal GbE |
| Power | Main battery 12V rail | Main battery 12V rail |
| Cooling | Independent heatsink + fan | Independent heatsink + fan |

## 4.3 SBC Interconnect Bus

+-----------------------------------------------------+
    |            Hub UGV (internal layout)                |
    |                                                       |
    |  +--------------+ GbE    +--------------+            |
    |  |   SBC #1     |<------>|   SBC #2     |            |
    |  |   (SLAM)     |        |  (Comm/Video)|            |
    |  |              |        |              |            |
    |  | ROS2 nodes:  |        | ROS2 nodes:  |            |
    |  | - slam_fuse  |        | - mesh_mon   |            |
    |  | - map_pub    |        | - lte_gw     |            |
    |  |              |        | - gst_relay  |            |
    |  +--------------+        +--------------+            |
    |                              |                        |
    |                              v                        |
    |  Wi-Fi 6 Mesh --- LTE --- Other Robots / Operator     |
    +-----------------------------------------------------+

## 4.4 SBC Failure Response

| Failure | Behavior | Mission Impact |
| --- | --- | --- |
| SBC #1 (SLAM) failed | Aggregated SLAM suspended, each follower runs local SLAM | Mission continues (no global map) |
| SBC #2 (Comm) failed | Other Wi-Fi6 mesh robots assist routing | Mission pause or SBC reboot |
| Both SBCs nominal | All functions | Full |

# 5. SLAM Aggregated Collection

## 5.1 Period Change Rationale

| Aspect | v1.0 (1 Hz) | v1.1 (5s aggregated (v1.3)) |
| --- | --- | --- |
| Comm load (9 robots) | 90-360 KB/s | 1-3 KB/s (1/100 reduction) |
| Hub SBC #1 load | Continuous 100% busy | Periodic 10-20% busy |
| Global map refresh | 1s (excessive) | **5s** (v1.3) (sufficient) |
| Real-time avoidance | Global map dependent | Local SLAM/Nav2 (proper) |
| Bandwidth headroom | Low | Adequate for 1s prediction |

## 5.2 SLAM Data Flow

[Each Follower] Local SLAM (slam_toolbox, 1Hz local)
          |
          | Every **5s** (v1.3) send aggregated occupancy grid delta (SLAMLocalDelta)
          v
    [Hub SBC #1] multirobot_map_merge
          | - feature-based merge (ORB/RANSAC)
          | - pose-graph optimization (g2o)
          v
    [Hub SBC #1] maintains global merged map
          |
          | Every **5s** (v1.3) broadcast (SLAMAggregatedMap, or delta only)
          v
    [All Robots] Nav2 cost map static layer update

## 5.3 Dynamic Period Adjustment

| Operational Situation | Recommended | Rationale |
| --- | --- | --- |
| Wide-area recon | 60s | Slow-changing terrain |
| General (default) | 30s | Balanced |
| Urban/indoor | 15s | Complex terrain |
| After formation change | Immediate (1x) | Re-align with new formation |
| Heavy obstacles | 10s temp | Safety |

## 5.4 Comm Load Verification

Per aggregated bundle in 8-robot squadron:

- Occupancy grid (100×100m, 0.1m resolution) = 1000 × 1000 cells = ~1 MB raw

- After PNG compression: ~50-200 KB

- 9 robots concurrent 30s: avg 30 KB/s, worst 60 KB/s

- Wi-Fi 6 Mesh capacity (12.5-37.5 MB/s) used < 1%

## 5.5 Local vs Global SLAM Role Separation

| Role | Local (each robot) | Global (Hub aggregated) |
| --- | --- | --- |
| Update rate | 1 Hz | **5s** (v1.3) |
| Purpose | Real-time avoidance, Nav2 | Long-term recon, global planner |
| Coverage | ~30m around robot | Hundreds of meters - km |
| Storage | Each robot | Hub UGV SBC #1 |
| Loss impact | Reduced avoidance capability | Mission planning info loss |

# 6. Video Streaming — GStreamer UDP/SRT

## 6.1 Background

v1.0 only specified "LTE backhaul, BEST_EFFORT". v1.1 confirms:

- Encoder: H.265 (default) / H.264 (compatibility)

- Container: MPEG-TS

- Protocol: SRT (operator) / UDP (inter-robot)

- Pipeline: GStreamer

- Trigger: Android APP request (Pull mode)

## 6.2 SRT vs UDP Selection

| Aspect | UDP (Plain) | SRT (adopted, operator) |
| --- | --- | --- |
| Latency | Lowest (~50ms) | 200-500ms (buffer) |
| Packet loss | None (glitches) | Auto-retransmit (ARQ) |
| Encryption | Separate impl | Built-in AES-128/256 |
| Bandwidth adapt | None | Dynamic bitrate |
| Firewall traversal | Difficult | Easy (1 port) |
| Recommended | Inter-robot light | Operator screen |
| KPP 200ms met | Met | Met with live latency=120ms |

## 6.3 GStreamer Pipelines

### 6.3.1 Sender (Follower or Hub) — H.265 + SRT

gst-launch-1.0 -v \
        v4l2src device=/dev/video0 \
        ! video/x-raw,width=1920,height=1080,framerate=30/1 \
        ! x265enc bitrate=2000 tune=zerolatency speed-preset=ultrafast \
        ! h265parse \
        ! mpegtsmux \
        ! srtsink uri="srt://<operator_ip>:8888?mode=caller&latency=120&passphrase=..."

### 6.3.2 UDP Variant (inter-robot)

gst-launch-1.0 -v \
        v4l2src device=/dev/video0 \
        ! video/x-raw,width=640,height=480,framerate=15/1 \
        ! x264enc bitrate=500 tune=zerolatency \
        ! h264parse \
        ! mpegtsmux \
        ! udpsink host=<hub_ip> port=5000

### 6.3.3 Hub Relay (Forwarding)

gst-launch-1.0 -v \
        udpsrc port=5000 caps="application/x-rtp" \
        ! rtpmp2tdepay \
        ! tsdemux \
        ! srtsink uri="srt://<operator_ip>:8888?mode=caller&latency=120"

### 6.3.4 Receiver (Android APP)

// Android (ExoPlayer)
    val srtUri = Uri.parse("srt://<robot_ip>:8888?mode=listener&latency=120")
    val source = ProgressiveMediaSource.Factory(dataSourceFactory)
        .createMediaSource(MediaItem.fromUri(srtUri))
    exoPlayer.setMediaSource(source)
    exoPlayer.prepare()
    exoPlayer.playWhenReady = true

## 6.4 Video Request Flow (Pull mode)

[Android APP]                [Hub UGV]              [Target Follower]
         |                          |                        |
         | 1. VideoStreamRequest    |                        |
         ------------------------->|                        |
         | {target_id, srt, h265}   |                        |
         |                          | 2. start_stream cmd    |
         |                          ----------------------->|
         |                          |                        |
         |                          | 3. SRT stream (forward)| stream start
         |                          |<-----------------------
         | 4. VideoStreamHandle     |                        |
         |<-------------------------                        |
         | {srt_uri, key}           |                        |
         |                          |                        |
         | 5. SRT receive & decode  |                        |
         |<================================================<| (relay)
         |                          |                        |
         | 6. VideoStreamStop       |                        |
         ------------------------->| 7. stop_stream         |
         |                          ----------------------->| stream stop

## 6.5 Bitrate and Quality Policy

| Quality | Resolution | FPS | Bitrate | Use |
| --- | --- | --- | --- | --- |
| HD | 1920x1080 | 30 fps | 3-5 Mbps | Detailed recon, target ID |
| FHD | 1280x720 | 30 fps | 1.5-2 Mbps | General (default) |
| Low | 640x480 | 15 fps | 300-500 Kbps | Comm-constrained |
| Thumbnail | 320x240 | 5 fps | 100 Kbps | Multi-screen monitoring |

## 6.6 Security

- SRT built-in AES-128 (performance-balanced)

- Passphrase: per-mission auto-issuance (DDS Security key mgmt)

- Hub-Operator segment: IPsec or OpenVPN tunnel

- Robot-Hub segment (LAN): WPA3 + DDS Security

# 7. Additional Message Definitions

Messages defined in this SDD are formally integrated in SAN-IDS-CMD-001 v1.1. This section summarizes.

## 7.1 SurveillanceSectorAssignment (new)

message SurveillanceSectorAssignment {
        uint32  sequence              = 1;
        uint32  robot_id              = 2;
        float   sector_start_deg      = 3;
        float   sector_end_deg        = 4;
        uint32  valid_period_sec      = 5;
        string  priority              = 6;
        string  mode_hint             = 7;
        uint64  timestamp_ms          = 8;
    }
    // QoS: P1, RELIABLE, 10s period + event

## 7.2 PanTiltCommand (new)

message PanTiltCommand {
        uint32  robot_id            = 1;
        float   target_pan_deg      = 2;
        float   target_tilt_deg     = 3;
        float   speed_dps           = 4;
        string  mode                = 5;
        float   sweep_range_deg     = 6;
        uint64  timestamp_ms        = 7;
    }
    // QoS: P1, RELIABLE

## 7.3 SLAMAggregatedMap (new)

message SLAMAggregatedMap {
        bytes   occupancy_grid_png  = 1;
        Pose2D  origin              = 2;
        float   resolution_m        = 3;
        uint32  width_cells         = 4;
        uint32  height_cells         = 5;
        uint32  contributing_robots = 6;
        uint64  timestamp_ms        = 7;
    }
    // QoS: P2, RELIABLE, TRANSIENT_LOCAL, **5s** (v1.3)

## 7.4 SLAMLocalDelta (modified — replaces SLAMDelta)

message SLAMLocalDelta {
        string  robot_id                  = 1;
        bytes   occupancy_grid_delta_png  = 2;
        Pose2D  origin                    = 3;
        float   resolution_m              = 4;
        uint64  coverage_start_ms         = 5;
        uint64  coverage_end_ms           = 6;
        uint64  timestamp_ms              = 7;
    }
    // QoS: P2, RELIABLE, **5s** (v1.3)

## 7.5 VideoStreamRequest (new)

message VideoStreamRequest {
        uint32  target_robot_id  = 1;
        string  protocol         = 2;
        string  codec            = 3;
        string  quality          = 4;
        bool    encryption       = 5;
        string  action           = 6;
        uint64  timestamp_ms     = 7;
    }
    // QoS: P1, RELIABLE

## 7.6 VideoStreamHandle (modified)

message VideoStreamHandle {
        uint32  target_robot_id     = 1;
        string  srt_uri             = 2;
        string  codec               = 3;
        string  quality             = 4;
        uint32  actual_bitrate_kbps = 5;
        string  passphrase          = 6;
        string  status              = 7;
        uint64  timestamp_ms        = 8;
    }
    // QoS: P1, RELIABLE

# 8. Existing SDD Impact

| Unified SDD § | Existing Spec | Impact | Action |
| --- | --- | --- | --- |
| §3 (Hub platform) | Single SBC | Changed to dual SBC | v1.1 absorbed |
| §9 (SLAM/comm) | Follower -> Hub 1 Hz | 5s aggregated (v1.3) | v1.1 absorbed |
| §14 (Message) | 31 messages | 6 messages affected | IDS v1.1 absorbed |

## 8.1 TST Test Scenario Impact

| New ID | Verification Target | Test Environment |
| --- | --- | --- |
| S15-1 | 8-robot 360° surveillance accuracy | Gazebo W5 |
| S15-2 | Pan-tilt sweep detection rate | Gazebo W2 |
| S15-3 | Hub SBC #1/#2 partial operation | Gazebo W1 |
| S15-4 | SLAM 30/60s global map consistency | Gazebo W3 |
| S15-5 | GStreamer SRT 200ms latency KPP | Real robot (LTE) |
| S15-6 | Multi-follower simultaneous video | Real robot (operator APP) |

# 9. Conclusion

This Supplementary SDD formally specifies the following four design decisions made in the SAN-SDD-SWARM-001 operational phase:

- 8-robot 360° surveillance distribution (Leader 10s period, Follower front 180°, Hub rear 180°)

- Follower Pan-tilt dynamic sweep (30°/sec during driving)

- Hub UGV dual SBC (SLAM-dedicated + Comm/Video-dedicated)

- SLAM **5s** (v1.3) collection + GStreamer SRT video + Android pull

The design improves operability and performance without changing v1.0 essence. Relaxing SLAM period frees bandwidth for 1s-prediction broadcast and video streaming.