Squad Swarm Operating Procedure

SOP Card

8-Robot Heterogeneous Swarm — For Operator Use

문서번호 SAN-OPS-SOP-001

v1.5

㈜스카이오토넷 / 군집제어형 소형 전술로봇 사업단

| Document ID | SAN-OPS-SOP-001 |
| --- | --- |
| Version | v1.5 |
| Classification | Standard Operating Procedure |
| Preceding Doc | SAN-SDD-SWARM-001 v1.5 |
| Security | Internal use |

# Revision History

| Version | Changes |
| --- | --- |
| v1.0 | Initial release. Core rules + presets + modes + Tier colors + emergency procedures. |
| v1.1 | Section 7 FAQ added (surveillance distribution, video viewing, Hub UGV dual SBC). |
| v1.2 | PM decisions D1~D7 + A1~A5 applied: (D1) 8-robot squadron (Leader S1 + Hub S2 + F1-F6 = S3-S8); (A1) Core rule #4 updated — production/demo/**bench** yaml overlay modes; (A2) Section 8 Demo procedure (DEMO 6-phase) added; (A3) command_id echo pattern guidance; (A4) Node watchdog 3s policy. |
| v1.4 | Deputy UGV + Limp Mode operational guidance: (1) Section 7 FAQ — Deputy UGV role, Leader succession priorities; (2) Section 7 FAQ — Limp Mode operational restrictions (no fire, RTB recommended); (3) Squadron composition updated (4~8 variable). |
| v1.5 | DCN-2026-001 applied (2026-05-12): (1) §7.11 Limp Mode FAQ NEW — fire authorization policy aligned with SDD §5.7.2 (Option A: fire RETAINED with HMAC-SHA256 + Two-key arming + red banner + audit log). Earlier v1.4 "no fire, RTB recommended" stance OFFICIALLY SUPERSEDED; (2) Rule #4 deployment_mode 3 → 5 types (production / demo / lab_test / bench / development); (3) Related Docs version pins v1.4 → v1.5. |

# Table of Contents

# 1. Core Operational Rules (Mandatory Knowledge)

| # | Rule |
| --- | --- |
| 1 | On "Assault" command, d = 15m applied automatically — no threat level judgment required |
| 2 | Leader (Go2) auto-limited to 1.3 m/s — never command higher speed or overtaking |
| 3 | Near cliffs/ravines, auto-transition to single column — override available within 5s |
| 4 | deployment_mode yaml overlay 5 types: production(live) / demo(show, weapons SIMULATED) / lab_test(real HW partial, weapons DENIED) / bench(internal PC test, weapons DENIED) / development(developer debug only) — bench/demo/lab_test/development never in live operations. Only production permits live fire |
| 5 | Fire authorization (Fire Confirm) requires operator touch input — no auto-fire |
| 6 | Leader battery < 30% triggers Hub UGV auto-takeover — mission continues |
| 7 | v1.1 NEW: Camera surveillance area auto-assigned by Leader every 10s — no operator action needed |
| 8 | v1.1 NEW: View Video by tapping robot icon on map → "View Video" (Pull mode) |

# 2. Operational Presets (Android APP → Formation Select)

| Preset | d | θ | Use | Selection Criterion |
| --- | --- | --- | --- | --- |
| Narrow Penetration | 3 m | 40° | Narrow corridor, woods, alley | Passage width < 6m |
| Recon/Defense | 5 m | 90° | 360° viewport, flank perimeter | Recon, patrol, base defense |
| Wide Recon | 7 m | 120° | Distributed surveillance | Open terrain wide recon |
| Assault | 15 m | 60° | Forward firepower + shell evasion | Enemy contact / engagement / breakthrough |

# 3. Operating Modes (Android APP → Mode Select)

| Mode | Auto Flow | Operator Action |
| --- | --- | --- |
| Offensive | Assembly → Column → V → Assault → Rank → Merge → Column | "Assault" button tap, "Confirm Fire" on engagement |
| Defensive | Diamond → Drone detection → Auto share → Neutralize | "Defense" button tap, position designate then monitor |
| Return (RTH) | Auto convert to single column, return to home | "Return" button tap or auto (battery threshold) |

# 4. Follower Status Colors (Map Screen)

| Color | Tier | Meaning | Operator Action |
| --- | --- | --- | --- |
| Green | T0 Predictive Track | Normal (1s prediction position tracking) | None (normal) |
| Cyan | T1.5 Auto Reroute | Local obstacle ±2m detour in progress | None (auto-recovery) |
| Yellow-Green | T1 Normal | Body-frame offset tracking (no prediction) | Check comm status |
| Yellow | T2 Catch-Up | Lagging, 1.2x speed | Consider lowering Leader speed |
| Orange | T3 Hard Catch-Up | Max speed pursuit | Consider pause, check comm |
| Red | T4 Breadcrumb (blinking) | Returning via Leader past path — DANGER | Halt, situation review, RTH if needed |

# 5. Emergency Procedures

## 5.1 Communication Loss (No Leader Response)

- Within 30s: Followers self-handle via Tier 1/2/3. No action needed.

- > 30s: Auto T4 Breadcrumb entry. All followers display blinking.

- > 60s: Auto RTH (return to home). "Cancel RTH" button overrides.

- LoRa emergency channel allows emergency commands (stop, return).

## 5.2 Drone Threat Detection (RF Scanner Alert)

- Auto alert screen + drone bearing/distance display

- Within 1km: "Jam" button → Soft-kill auto (operator confirm)

- Within 150m: "Confirm Fire" button → Hard-kill firing (gyro stabilized)

## 5.3 Enemy Personnel Detection (AI Object Detection)

- "Potential Threat" yellow marker + distance/bearing auto-calculation

- Target tap → wind/trajectory-adjusted "Predicted Hit Point" display

- "Confirm Fire" → gyro stabilized + precision strike (within 300m)

## 5.4 Leader Loss (Battery / Damage / Comm Loss)

- Battery < 30%: Hub UGV auto-takeover (5s alert)

- Damage/full comm loss: Modified Raft priority succession (Hub UGV 1st)

- Reform time approximately 5-8s. "Re-organizing" displayed.

- On new leader decision, mission auto-continues.

## 5.5 GPS Jamming (RTK Fix Loss)

- 0-5s: Dead-reckoning short-term hold (formation stable)

- 5-30s: LiDAR SLAM augmentation (Robosense E1)

- > 30s: Tier 4 autonomous mode. Return to safe area.

- Operator: acknowledge "GPS Loss" warning, prefer halt command

# 6. Quick Decision Reference

| Situation | Recommended Action |
| --- | --- |
| Mountain/valley march | Auto single column. Accept (or no action) |
| Urban alley passage | Select "Narrow Penetration" preset (d=3m) |
| Pre-contact (need visibility) | "Recon/Defense" (d=5m, θ=90°) or "Wide Recon" (d=7m, θ=120°) |
| Enemy ID → Assault order | "Assault" button (auto d=15m, no threat judgment) |
| Base 360° perimeter | "Defensive Mode" + "Recon/Defense" preset (auto diamond) |
| Multi-drone simultaneous intrusion | Auto free-spread entry, verify "Auto Share" ON |
| Low battery alert | "Return (RTH)" button (auto return to home) |

# 7. v1.1 FAQ — Camera / Video / SLAM

## 7.1 Q: How is camera surveillance area determined?

**A:** The Leader robot auto-distributes sectors every 10s. No operator action required.

| Robot | ID | Coverage | Operation |
| --- | --- | --- | --- |
| Leader (Go2) | S1 | Front 0° ±30° | Fixed (camera 1x Wide, HFOV 60°) |
| Hub UGV | S2 | Rear ±90° (180°) | Pan-tilt sweep 30°/sec (8s period) |
| F1 / F2 | S3/S4 | Front-left/right 50° each | Pan-tilt sweep 30°/sec |
| F3 / F4 | S5/S6 | Left/right flank 50° each | Pan-tilt sweep |
| F5 / F6 | S7/S8 | Left/right rear 50° each | Pan-tilt sweep |

On threat detection, the Leader immediately reassigns sectors to focus followers on the threat direction. On follower loss, remaining followers auto-redistribute.

## 7.2 Q: I want to view video from a specific robot.

**A:** Tap the robot icon on the map → select "View Video" menu (Pull mode).

- Tap robot icon on map

- Select "View Video" from popup menu

- Quality selection: High (HD) / Standard (FHD, default) / Low / Thumbnail

- Hub UGV starts GStreamer SRT pipeline (display within 200ms)

- To end: Tap "Stop View Video" → auto cleanup

**Multiple videos**: If 4+ requested simultaneously, auto-downgrade to "Thumbnail" mode (320x240). Up to 9 robot videos monitorable simultaneously.

## 7.3 Q: "Hub UGV Comm SBC Failure" alert appeared.

**A:** The Hub UGV has dual SBC architecture, allowing partial operation when one fails.

| Alert | Cause | Mission Impact | Action |
| --- | --- | --- | --- |
| "SLAM SBC Failure" | Hub SBC #1 failure | Global map update suspended, local SLAM OK | Mission continues, inspect after return |
| "Comm SBC Failure" | Hub SBC #2 failure | LTE/video stream suspended, mesh OK | Video viewing unavailable, mission continues |
| "Both SBCs Failure" | Both SBCs failed (rare) | Leader succession risk | Immediate RTH consideration |

## 7.4 Q: Global map refreshes every 30-60s — is this normal?

**A:** From v1.1, SLAM aggregated bundle refreshes every 30s (default). Rationale for not 1-second real-time refresh:

- Real-time avoidance handled by each robot's local SLAM at 1Hz (immediate avoidance)

- Global map is for mission planning/recon — 30s refresh sufficient

- 100x comm load reduction frees bandwidth for 1s prediction broadcast

PM may adjust period based on operation (urban: 15s, wide recon: 60s).

## 7.5 Q: What LiDAR models are used?

| Robot | ID | LiDAR Model | Characteristics |
| --- | --- | --- | --- |
| Leader (Go2) | S1 | Unitree L1 (built-in) | 360° rotating, 30m range |
| Hub UGV | S2 | Robosense E1 (solid-state) | 200m range, 120° x 90° FoV (v1.3 correction) |
| Follower UGV | S3-S8 | Robosense E1 (solid-state) | 200m range, 120° FoV (6 units) |

Robosense E1 is a solid-state LiDAR with no moving parts, robust against military vibration/shock. Capable of long-range 200m reconnaissance and effective when combined with RF scanner cross-validation.

## 7.6 Q: How does it operate in weak Wi-Fi environments?

**A:** OpenWrt 24.10 Wi-Fi 6 Mesh maintains connectivity via multi-path.

- Mesh node auto-routing (batman-adv BATMAN V)

- AP steering DAWN auto-selects optimal AP

- On Wi-Fi loss, mwan3 auto-switches to LTE backhaul within 8s

- Last resort: LoRa emergency channel (stop, return commands only)

## 7.7 Q: If enemy detected during sweep?

**A:** Auto-transition to Track mode, Pan-tilt locks on target. Operator action:

- "Potential Threat" yellow marker auto-displayed

- Leader auto-reassigns nearby 2-3 followers' sectors to threat direction (cross-cueing)

- Operator: tap to confirm target

- Select Confirm Fire or "Observe Only"

## 7.8 Q: How is automatic avoidance determined? (v1.3 new)

**A:** Each UGV computes a Local Cost Map (obstacle map) on its own SBC every second. If hazard cells exist on the predicted path, Tier 1.5 is triggered and an automatic +/-2m lateral detour is performed.

| Condition | Cost Map class | Action |
| --- | --- | --- |
| Obstacle height >= 235 mm | lethal (cost 254) | **Mandatory detour** (+/-2m lateral) |
| Ditch width >= 220 mm | lethal | Mandatory detour |
| Slope > 30 deg | lethal | Mandatory detour |
| Obstacle 100-235 mm | inflated (cost 200) | Detour if possible, slow pass |
| Slope 15-30 deg | cautious (cost 100) | Slow pass |
| Slope < 15 deg, obstacle < 100 mm | free (cost 0) | Normal pass |

After **3 consecutive avoidance failures**, an alert "Manual intervention required" appears on the operator screen.

## 7.9 Q: "LTE Backup Activated" alert appeared (v1.3 new)

**A:** When Hub UGV (S2) LTE modem is in a non-operational state, Follower S3 automatically takes over the LTE gateway role. The mission continues, and telemetry remains normal (up to 30 sec temporary gap is possible).

| Status Display | Meaning | Recommended Action |
| --- | --- | --- |
| "Hub LTE Normal" (Green) | Hub S2 operating LTE gateway | Normal — no action |
| "LTE Backup Active — S3" (Yellow) | S3 took over LTE backup (Hub LTE down) | Continue mission, inspect Hub LTE after return |
| "LTE Total Outage" (Red) | Hub + S3 LTE both non-operational | Operate on Wi-Fi 6 mesh only, approach operator |
| "Hub LTE Recovered — S3 demoted" (Green) | Hub LTE recovered, normal distribution | Normal — verify split-brain prevented |

On LTE backup follower (S3) failure, auto-chain promotion to next candidate (S5). Backup activation threshold time <= 10s.

## 7.10 Q: Is the SLAM global map updated every 5 seconds? (v1.3 updated)

**A:** From v1.3, the Hub UGV SLAM aggregation period is shortened from **30-60s -> 5 sec**. This is to maintain consistency with the Cost Map (1Hz). The operator screen's global map now refreshes every 5 seconds.

- Real-time avoidance uses Local SLAM + Cost Map (1Hz) — instant action

- Global map reconnaissance info refreshes at 5-second intervals

- Can be shortened to 3 sec by PM decision for urban/indoor penetration

## 7.11 Q: Limp Mode entered. Is fire still available? (★ v1.5 policy confirmed — DCN-2026-001 D-004)

A: YES, fire is RETAINED. Even when Hub UGV (S2) and Deputy UGV (S3) are both inoperative and Limp Mode is entered, fire/strike commands remain available under the following safeguards (SDD §5.7.2 / DCN-2026-001 D-004 Option A confirmed). This is the official policy; any conservative reading in earlier v1.4 drafts is hereby superseded.

| Item | Limp Mode behavior |
| --- | --- |
| Operator control | ✅ Android app connects directly to surviving robots via Wi-Fi 6 Mesh |
| Fire authentication | ✅ HMAC-SHA256 with mesh shared secret (LTE bypass) |
| Two-key arming | ✅ Target tap (1st key) + "Confirm Fire" red button (2nd key) — same as production mode |
| Red banner | ✅ Always displayed at top: "LIMP MODE — fire permitted, situation awareness reduced" |
| Autonomous engagement | ❌ AI auto-engagement / auto-tracking DISABLED (operator-initiated only) |
| Formation change / AI inference | ❌ Suspended (single-SBC processing burden) |
| Video viewing | ✅ Each robot sends GStreamer UDP/SRT directly to Android IP (no Hub relay) |
| Audit log | ✅ All Limp Mode fires tagged limp_mode_fire=true and recorded permanently |

Operational guidance: RTB (return-to-base) should be considered if mission permits, but if an imminent threat arises, exercise fire authority through normal Two-key procedure. Submit the audit log together with Hub/Deputy diagnostic data after mission completion.

Previous policy (some v1.4 drafts): "Limp Mode → fire prohibited, RTB recommended" was a conservative reading. In v1.5 this is OFFICIALLY SUPERSEDED. SDD §5.7.2 is the policy authority.

# 8. Demo Procedure (DEMO 6-Phase Sequencer)

## 8.1 Demo Mode Entry Conditions

- deployment_mode = `demo` (yaml overlay applied)

- Weapons auto-disabled (gun_trigger_permission = DENIED forced)

- BB pellets / blanks or dummy mode operation

- Operator taps "Start Demo" button to initiate auto-sequence

## 8.2 DEMO 6-Phase Auto Sequence

| Phase | Action | Default Params | Operator |
| --- | --- | --- | --- |
| DEMO_PHASE_IDLE | Wait for sequence trigger | — | Tap Start Demo |
| DEMO_PHASE_FORWARD | Advance demo_forward_distance_m | Default 5 m, 0.5 m/s | Observe |
| DEMO_PHASE_SCAN | Auto scan specified pan/tilt range | pan ±60°, tilt -10° to +5° | Auto advance on target detect |
| DEMO_PHASE_ENGAGE | Lock target → fire demo_fire_duration_sec | Default 2 sec, BB/dummy | Observe (auto-fire alert) |
| DEMO_PHASE_REVERSE | Reverse demo_reverse_distance_m | Default 5 m, 0.5 m/s | Observe |
| DEMO_PHASE_COMPLETE | Return to IDLE, show result screen | — | Tap Confirm |

## 8.3 Forced Stop / Resume

- Top-right "Stop Demo" button → immediate IDLE return (resetDemoSequence)

- EmergencyStop input immediately halts SCAN/ENGAGE phase

- After stop: "Restart Demo" available (begins again from FORWARD)

## 8.4 Recommended Demo Environment

- Indoor demo: Narrow Penetration preset (d=3m), Leader Go2 + Hub UGV + 2-3 followers

- Outdoor (small venue): Recon/Defense preset (d=5m), Leader + Hub + 4 followers

- DAPA/VIP formal demo: Full 8 robots, Wide Recon preset

## 8.5 Operator Demo Display

- Current Phase displayed in large text (FORWARD / SCAN / ENGAGE / REVERSE)

- 8-robot positions + sectors + target markers in real time

- "Demo Mode" label persistently displayed (yellow banner)

- `deployment_mode=demo, weapons disabled` status displayed

# 9. Appendix

## 9.1 Color Guide (Map Markers)

| Color | Meaning |
| --- | --- |
| Green | Normal follower / surveillance area |
| Blue | Leader (Go2) / protected area |
| Purple | Hub UGV / comm relay area |
| Yellow | Potential threat / Tier 2-3 follower |
| Red | Confirmed threat / enemy / Tier 4 follower |
| Rainbow | Multi-sector distribution (rotates every 10s) |

## 9.2 Emergency Contact

On suspected robot loss/capture: "Compromise" button → immediate certificate revocation for that robot. CRL broadcasts to entire swarm, making that robot unusable.

## 9.3 Related Documents

- SAN-SDD-SWARM-001 v1.5 — Unified SDD (detailed design)

- SAN-SDD-SUR-001 v1.4 — Surveillance/SLAM/Video Supplementary SDD

- SAN-IDS-CMD-001 v1.4 — Interface Definition Specification

─ This SOP is an operational summary of SAN-SDD-SWARM-001 v1.4. Refer to Unified SDD for details ─