인터페이스 정의 명세서

Interface Definition Specification

8대 군집 — 35 메시지 + command_id echo 패턴

문서번호 SAN-IDS-CMD-001

v1.2

㈜스카이오토넷 / 군집제어형 소형 전술로봇 사업단

| 문서 ID | SAN-IDS-CMD-001 |
| --- | --- |
| 버전 | v1.4 |
| 문서 분류 | Interface Definition Specification |
| 선행 문서 | SAN-SDD-SWARM-001 v1.1 |
| 메시지 형식 | Google Protobuf v3 + ROS 2 .msg |
| 전송 계층 | Wi-Fi 6 Mesh (OpenWrt) + 군용 LTE + LoRa |
| 메시지 총합 | 35 (필드 보강: command_id echo) |
| 편대 구성 | 8대 (Leader S1 + Hub UGV S2 + Follower S3-S8) |

# 개정 이력

| 버전 | 변경 사항 |
| --- | --- |
| v1.0 | 최초 발행. 31 표준 메시지 (8 명령 + 9 텔레메트리 + 9 로봇간 + 5 인프라). |
| v1.1 | v1.0의 31 메시지 모두 유지하며 다음 변경: (a) §3에 VideoStreamRequest 신규 (Android pull 방식 영상 요청); (b) §4의 VideoStreamHandle을 GStreamer SRT/UDP 구체화하여 수정; (c) §5에 SurveillanceSectorAssignment, PanTiltCommand, SLAMAggregatedMap 신규 (3종); (d) §5의 기존 SLAMDelta를 SLAMLocalDelta로 대체 (수집 주기 1Hz → 5초 (v1.3 단축) 통합본). 총합 35 메시지. |
| v1.2 | PM 결정 A3 (command_id echo 패턴) 반영. 주요 명령 메시지 8종에 command_id (uint32) 필드 추가, RobotStatus + SwarmHealthSummary에 last_received_command_id echo 필드 추가. 메시지 총 개수 동일(35), schema minor bump. 또한 D2 LeaderElection priority chain 명시 (Hub UGV 최우선). |
| v1.4 | Deputy UGV 메시지 필드 + 신규 메시지: (1) LeaderRoleAnnouncement 메시지 신규 (§5.15) — 4-tier Leader 승계; (2) HubRoleAnnouncement 메시지 신규 (§5.16) — Hub-Deputy 이중화; (3) RobotStatus에 is_deputy_ugv, battery_percent 필드 추가; (4) deputy_chain 기본값 갱신 — [3, 2, ...] (Deputy 1순위). |

# 목차

# 1. 개요

## 1.1 본 문서의 목적과 범위

본 문서는 본 사업 군집 시스템의 모든 통신 메시지를 단일 명세서에 정의한다. 적용 범위:

- 운용병사 Android 앱 → Leader 로봇 (명령 하달)

- Leader 로봇 → 운용병사 Android 앱 (상태·텔레메트리)

- Leader → Follower (대형·예측 위치·감시 sector)

- Follower → Leader/Hub (상태·SLAM 통합본)

- Hub UGV ↔ 광역 C2 (군용 LTE 백홀)

## 1.2 메시지 카테고리

| 카테고리 | 메시지 수 | 전송 방향 |
| --- | --- | --- |
| §3 운용병사 명령 | 9 (+1 신규) | Tablet → Leader |
| §4 군집 텔레메트리 | 9 (1 수정) | Swarm → Tablet |
| §5 로봇 간 | 12 (+3 신규, 1 수정) | Leader ↔ Follower ↔ Hub |
| §6 인프라/시스템 | 5 | 양방향 |
| 총합 | 35 | — |

# 2. 메시지 아키텍처

## 2.1 Topic 명명규칙

/{robot_namespace}/{category}/{message_type}
    예시:
    /leader/cmd/formation              # 운용병사 → leader
    /leader/cmd/surveillance_sectors    # 운용병사 → leader (영역 설정)
    /swarm/predict/follower_target      # leader broadcast
    /swarm/surveillance/sector_assign   # leader → 모든 follower (10s 주기)
    /hub/slam/aggregated_map            # Hub broadcast (5초 (v1.3 단축))
    /{robot_id}/cmd/pantilt              # leader 또는 operator → robot
    /tablet/cmd/video_request            # tablet → hub

## 2.2 Namespace 정책

| Namespace | 용도 | 예시 topic |
| --- | --- | --- |
| /leader | Leader (Go2) 전용 | /leader/status, /leader/cmd |
| /hub | Hub UGV 전용 | /hub/slam, /hub/video_forward |
| /follower{N} | 각 follower (N=1..7) | /follower1/status |
| /swarm | 전체 broadcast | /swarm/predict, /swarm/surveillance |
| /tablet | 운용병사 앱 | /tablet/heartbeat |

## 2.3 QoS 4-Tier

| 우선순위 | Reliability | Durability | History | Deadline | 사용 메시지 예 |
| --- | --- | --- | --- | --- | --- |
| P0 Critical | RELIABLE | TRANSIENT_LOCAL | KEEP_LAST(10) | 100 ms | FormationCommand, EmergencyStop |
| P1 Tactical | BEST_EFFORT/REL | VOLATILE | KEEP_LAST(3) | 200 ms | RobotStatus, FollowerTarget |
| P2 Operational | RELIABLE | TRANSIENT_LOCAL | KEEP_LAST(5) | 1 s | SLAMLocalDelta, SLAMAggregatedMap |
| P3 Background | BEST_EFFORT | VOLATILE | KEEP_LAST(1) | 5 s | LogEvent |

# 3. 운용병사 명령 메시지 (Tablet → Leader/Hub)

## 3.1 FormationCommand

| 속성 | 값 |
| --- | --- |
| Direction | Tablet → Leader |
| Topic | /leader/cmd/formation |
| QoS | P0, RELIABLE, KEEP_LAST(10) |
| Rate | 이벤트 |

message FormationCommand {
        uint32         command_id       = 1;   // ★ v1.2: ack echo용 (앱이 발급)
        enum FormationType {
            COLUMN=0; LINE=1; V_SHAPE=2; DIAMOND=3;
            ECHELON_LEFT=4; ECHELON_RIGHT=5; BOX=6;
            VEE_INVERTED=7; FREE_SPREAD=8;
        }
        FormationType  type             = 2;
        float          d                = 3;   // 1.5 ≤ d ≤ 20
        float          theta_deg        = 4;
        float          alpha_deg        = 5;
        float          transition_time  = 6;
        bool           dev_mode         = 7;
        uint64         timestamp_ms     = 8;
        string         requested_by     = 9;
    }

## 3.2 MissionStateCommand

message MissionStateCommand {
        uint32   command_id       = 1;   // ★ v1.2: ack echo
        enum State {
            IDLE=0; ASSEMBLY=1; OFFENSIVE=2; DEFENSIVE=3;
            PATROL=4; RTH=5; HOLD=6; EMERGENCY=7;
        }
        State    target_state     = 2;
        Pose2D   target_position  = 3;
        uint64   timestamp_ms     = 4;
        string   requested_by     = 5;
    }
    // QoS: P0, RELIABLE

## 3.3 FireAuthorization

message FireAuthorization {
        uint32    command_id          = 1;   // ★ v1.2: ack echo
        string    target_id           = 2;
        uint32    assigned_robot       = 3;
        enum FireMode {
            SINGLE_SHOT=0; THREE_BURST=1; FULL_AUTO=2; JAMMING=3;
        }
        FireMode  mode                = 4;
        Pose2D    target_pose         = 5;
        float     lead_distance_m     = 6;
        uint64    expires_at_ms       = 7;
        bytes     operator_signature  = 8;
        uint64    timestamp_ms        = 9;
    }
    // QoS: P0, RELIABLE, KEEP_ALL

## 3.4 JammingCommand

message JammingCommand {
        uint32  command_id              = 1;   // ★ v1.2: ack echo
        enum Band {
            BAND_2_4_GHZ=0; BAND_5_8_GHZ=1;
            BAND_GNSS_L1=2; BAND_GNSS_L2=3; BAND_ALL=4;
        }
        repeated Band  bands             = 2;
        Pose2D         target_direction  = 3;
        float          duration_s        = 4;
        float          power_w           = 5;
        uint64         timestamp_ms      = 6;
    }
    // QoS: P0, RELIABLE

## 3.5 WaypointCommand

message WaypointCommand {
        uint32          command_id        = 1;   // ★ v1.2: ack echo
        repeated Pose2D  waypoints         = 2;
        float            cruise_speed_mps  = 3;   // 0=auto, max 1.5
        bool             allow_replan      = 4;
        uint32           priority          = 5;
        uint64           timestamp_ms      = 6;
    }
    // QoS: P1, RELIABLE

## 3.6 ManualOverrideCommand

message ManualOverrideCommand {
        uint32  command_id      = 1;   // ★ v1.2: ack echo
        string  robot_id        = 2;
        enum Mode {
            DISENGAGE_AUTO=0; DIRECT_DRIVE=1; HOLD_POSITION=2; FOLLOW_ME=3;
        }
        Mode    mode             = 3;
        Velocity2D  manual_vel   = 4;
        uint64  expires_at_ms    = 5;
        uint64  timestamp_ms     = 6;
    }
    // QoS: P0, RELIABLE

## 3.7 EmergencyStop

message EmergencyStop {
        uint32    command_id       = 1;   // ★ v1.2: ack echo
        enum Severity {
            SOFT_STOP=0; HARD_STOP=1; DISARM=2; SHUTDOWN=3;
        }
        Severity  severity         = 2;
        string    reason           = 3;
        bytes     auth_signature   = 4;
        uint64    timestamp_ms     = 5;
    }
    // QoS: P0, RELIABLE, KEEP_ALL, LoRa fallback ON

## 3.8 OperatorHeartbeat

message OperatorHeartbeat {
        string    operator_id      = 1;
        bytes     auth_token       = 2;
        Pose2D    operator_pose    = 3;
        uint32    app_version      = 4;
        uint64    timestamp_ms     = 5;
    }
    // QoS: P1, BEST_EFFORT, 1 Hz

## 3.9 VideoStreamRequest (v1.1 신규)

Android APP가 영상 스트림을 요청하는 메시지. **Pull 방식**으로 불필요한 대역폭·전력 소비 방지.

| 속성 | 값 |
| --- | --- |
| Direction | Tablet → Hub UGV (SBC #2) |
| Topic | /tablet/cmd/video_request |
| QoS | P1, RELIABLE |
| Rate | 이벤트 (운용자 「영상 보기」 터치) |

message VideoStreamRequest {
        uint32  command_id          = 1;   // ★ v1.2: ack echo (sequence 대체)
        uint32  target_robot_id     = 2;
        string  protocol            = 3;   // "srt" | "udp"
        uint32  desired_port        = 4;
        string  codec               = 5;   // "h265" | "h264"
        string  quality             = 6;   // "hd" | "fhd" | "low" | "thumbnail"
        bool    encryption          = 7;
        string  passphrase_hint     = 8;
        string  action              = 9;   // "start" | "stop" | "change_quality"
        uint64  timestamp_ms        = 10;
    }

# 4. 군집 텔레메트리 메시지 (Swarm → Tablet)

## 4.1 RobotStatus

message RobotStatus {
        string  robot_id           = 1;
        enum Role { LEADER=0; HUB=1; FOLLOWER=2; }
        Role    role               = 2;
        Pose2D  pose               = 3;
        Velocity2D  velocity       = 4;
        float   heading_rad        = 5;
        enum Tier {
            T0_PREDICTIVE=0; T1_5_AUTO_REROUTE=1; T1_NORMAL=2;
            T2_CATCH_UP=3; T3_HARD_CATCH_UP=4; T4_BREADCRUMB=5;
        }
        Tier    current_tier       = 6;
        float   battery_percent    = 7;
        float   battery_voltage    = 8;
        float   cpu_load_percent   = 9;
        float   mem_used_percent   = 10;
        bool    rtk_fixed          = 11;
        bool    comm_healthy       = 12;
        bool    lte_active         = 13;   // v1.3: 이 robot이 현재 LTE 게이트웨이 역할
        bool    is_lte_backup_designated = 14;  // v1.3: backup 지정 follower 여부
        uint32  errors_active      = 13;
        uint32  last_received_command_id = 14;   // ★ v1.2: echo of last cmd
        uint64  timestamp_ms       = 15;
    }
    // QoS: P1, BEST_EFFORT, 1 Hz

## 4.2 FormationStatus

message FormationStatus {
        FormationType  active_type        = 1;
        float          d                  = 2;
        float          theta_deg          = 3;
        MissionState   mission_state      = 4;
        uint32         total_robots       = 5;
        uint32         healthy_robots     = 6;
        repeated string  unhealthy_ids    = 7;
        float          formation_error_m  = 8;
        bool           auto_column_active = 9;
        uint64         timestamp_ms       = 10;
    }
    // QoS: P1, BEST_EFFORT, 2 Hz

## 4.3 DetectedObject

message DetectedObject {
        string  object_id          = 1;
        string  detector_robot_id  = 2;
        enum ObjectClass {
            PERSON=0; VEHICLE=1; DRONE_FPV=2; DRONE_MULTI=3;
            DRONE_FIXED=4; UNKNOWN=5;
        }
        ObjectClass  obj_class    = 3;
        float        confidence   = 4;
        Pose2D       position     = 5;
        Velocity2D   velocity_estimate = 6;
        float        range_m      = 7;
        float        bearing_deg  = 8;
        bool         is_threat    = 9;
        uint32       threat_score = 10;
        bytes        thumbnail_jpeg = 11;
        uint64       detected_at_ms = 12;
    }
    // QoS: P0, RELIABLE, 이벤트

## 4.4 ThreatAlert

message ThreatAlert {
        string  alert_id          = 1;
        enum Severity {
            ADVISORY=0; CAUTION=1; WARNING=2; CRITICAL=3;
        }
        Severity  severity         = 2;
        enum ThreatType {
            DRONE_APPROACHING=0; ENEMY_PERSONNEL=1; ARTILLERY_DETECTED=2;
            AMBUSH_INDICATOR=3; COMM_JAMMING=4; GPS_JAMMING=5;
        }
        ThreatType  threat_type    = 3;
        string  related_object_id  = 4;
        float   time_to_impact_s   = 5;
        Pose2D  threat_position    = 6;
        string  message_ko         = 7;
        uint64  timestamp_ms       = 8;
    }
    // QoS: P0, RELIABLE, KEEP_ALL

## 4.5 TierStatusChange

message TierStatusChange {
        string    robot_id          = 1;
        Tier      from_tier         = 2;
        Tier      to_tier           = 3;
        string    reason            = 4;
        float     delta_m           = 5;
        uint64    timestamp_ms      = 6;
    }
    // QoS: P0, RELIABLE, 이벤트

## 4.6 FireResult

message FireResult {
        string  fire_id              = 1;
        string  shooter_robot_id     = 2;
        string  target_object_id     = 3;
        uint32  rounds_fired         = 4;
        bool    target_neutralized   = 5;
        float   hit_probability      = 6;
        bytes   post_image_jpeg      = 7;
        uint64  fired_at_ms          = 8;
    }
    // QoS: P0, RELIABLE

## 4.7 BatteryWarning

message BatteryWarning {
        string  robot_id           = 1;
        enum Level { LOW=0; CRITICAL=1; SHUTDOWN=2; }
        Level   level              = 2;
        float   percent            = 3;
        uint32  minutes_remaining  = 4;
        bool    should_rth         = 5;
        uint64  timestamp_ms       = 6;
    }
    // QoS: P0, RELIABLE

## 4.8 VideoStreamHandle (v1.1 수정 — GStreamer 구체화)

Hub UGV가 영상 스트림 핸들 정보를 APP에 반환. v1.0의 단순 RTSP URL → SRT/UDP URI + 암호화 키 + 실제 비트레이트로 확장.

message VideoStreamHandle {
        uint32  sequence            = 1;
        uint32  target_robot_id     = 2;
        string  srt_uri             = 3;   // e.g. "srt://hub_ip:8888?mode=listener"
        string  codec               = 4;   // "h265" | "h264"
        string  quality             = 5;   // "hd" | "fhd" | "low" | "thumbnail"
        uint32  actual_bitrate_kbps = 6;
        string  passphrase          = 7;   // 임무별 자동 발급
        string  status              = 8;   // "streaming" | "stopped" | "error"
        string  error_msg           = 9;
        uint64  stream_start_ms     = 10;
        uint64  timestamp_ms        = 11;
    }
    // QoS: P1, RELIABLE

## 4.9 SLAMUpdate

message SLAMUpdate {
        Pose2D  origin              = 1;
        float   resolution_m        = 2;
        uint32  width_cells         = 3;
        uint32  height_cells        = 4;
        bytes   occupancy_delta     = 5;
        uint32  contributing_robots = 6;
        uint64  timestamp_ms        = 7;
    }
    // QoS: P2, RELIABLE, 0.5 Hz (tablet 화면 갱신용)

# 5. 로봇 간 메시지 (Inter-Robot)

## 5.1 FollowerTargetMessage (1초 예측)

| 속성 | 값 |
| --- | --- |
| Direction | Leader → Follower (broadcast) |
| Topic | /swarm/predict/follower_target |
| QoS | P0, RELIABLE, KEEP_LAST(3) |
| Rate | 10 Hz (100 ms 주기) |
| 크기 | 약 200 byte |

message FollowerTargetMessage {
        uint32  follower_id            = 1;
        Pose2D  target_pose            = 2;
        uint64  valid_at_ms            = 3;
        repeated Pose2D  leader_path_segment = 4;
        uint32  sequence_num           = 5;
        float   confidence             = 6;
        Velocity2D  target_velocity    = 7;
        uint64  generated_at_ms        = 8;
    }
    message Pose2D {
        double  x        = 1;
        double  y        = 2;
        float   heading  = 3;
    }
    message Velocity2D {
        float   vx       = 1;
        float   vy       = 2;
        float   omega    = 3;
    }

## 5.2 BreadcrumbBroadcast

message BreadcrumbBroadcast {
        uint64  timestamp_us       = 1;
        Pose2D  position           = 2;
        float   speed_mps          = 3;
        enum TerrainClass {
            OPEN=0; FOREST=1; URBAN=2; NARROW=3; CLIFF=4;
        }
        TerrainClass  terrain        = 4;
        uint8   formation_id       = 5;
        uint32  sequence_num       = 6;
    }
    // QoS: P1, RELIABLE, KEEP_LAST(50), 0.5-2 Hz (1m 이동/0.5s)

## 5.3 LeaderElection (Modified Raft)

message LeaderElection {
        // D2 (v1.2): Hub UGV priority chain
        // Priority order: is_hub_ugv (S2 자동 최우선) > chain_index (S2→S3→...→S8) > battery > id
        enum MessageType {
            REQUEST_VOTE=0; VOTE_GRANTED=1; VOTE_DENIED=2;
            LEADER_ANNOUNCE=3; HEARTBEAT=4;
        }
        MessageType  type                = 1;
        uint32       term                = 2;
        string       candidate_id        = 3;
        float        candidate_priority  = 4;   // Higher = preferred (is_hub_ugv=infinity)
        bool         is_hub_ugv          = 5;   // ★ v1.2: explicit Hub flag
        uint32       chain_index         = 6;   // ★ v1.2: deputy chain position
        string       voter_id            = 7;
        uint64       timestamp_ms        = 8;
    }
    // QoS: P0, RELIABLE, KEEP_ALL, 이벤트 (Hub UGV priority 1순위)

## 5.4 EmergencyAlert

message EmergencyAlert {
        string  source_robot_id  = 1;
        enum AlertType {
            OBSTACLE_AHEAD=0; COLLISION_WARNING=1; CLIFF_DETECTED=2;
            FORMATION_BREAK=3; SENSOR_FAIL=4;
        }
        AlertType  type             = 2;
        Pose2D     alert_position   = 3;
        float      severity         = 4;
        string     description      = 5;
        uint64     timestamp_ms     = 6;
    }
    // QoS: P0, RELIABLE, 이벤트

## 5.5 SLAMLocalDelta (v1.1 수정 — 1Hz → 5초 (v1.3 단축) 통합본)

기존 SLAMDelta(1 Hz)를 본 메시지로 대체. 30-60초마다 누적 통합본 전송하여 통신 부하 1/100 감소.

message SLAMLocalDelta {
        uint32  sequence                  = 1;
        string  robot_id                  = 2;
        bytes   occupancy_grid_delta_png  = 3;   // PNG 압축
        Pose2D  origin                    = 4;
        float   resolution_m              = 5;
        uint64  coverage_start_ms         = 6;   // 통합본 시작 시각
        uint64  coverage_end_ms           = 7;   // 통합본 종료 시각
        uint64  timestamp_ms              = 8;
    }
    // QoS: P2, RELIABLE
    // 발신: 모든 robot (Leader + 7 Follower)
    // 수신: Hub UGV SBC #1
    // Rate: 30-60초 (default 30s, 동적 조정)

## 5.6 SLAMAggregatedMap (v1.1 신규)

Hub UGV SBC #1이 통합한 글로벌 맵을 broadcast하는 메시지. 모든 robot의 Nav2 cost map static layer를 갱신.

message SLAMAggregatedMap {
        uint32  sequence            = 1;
        bytes   occupancy_grid_png  = 2;   // PNG 압축 글로벌 맵
        Pose2D  origin              = 3;
        float   resolution_m        = 4;
        uint32  width_cells         = 5;
        uint32  height_cells        = 6;
        uint32  contributing_robots = 7;
        uint64  timestamp_ms        = 8;
    }
    // QoS: P2, RELIABLE, TRANSIENT_LOCAL
    // 발신: Hub UGV (SBC #1)
    // 수신: 모든 Robot + Operator Tablet
    // Rate: 30-60초 (동적 조정)

## 5.7 GlobalMapBroadcast (v1.0 유지, Operator 화면용)

message GlobalMapBroadcast {
        Pose2D  map_origin          = 1;
        float   resolution_m        = 2;
        uint32  width               = 3;
        uint32  height              = 4;
        bytes   full_map            = 5;
        bytes   delta_map           = 6;
        repeated string contributor_ids = 7;
        uint64  timestamp_ms        = 8;
    }
    // QoS: P2, RELIABLE, TRANSIENT_LOCAL, 0.5 Hz

## 5.8 HeartBeat

message HeartBeat {
        string  robot_id          = 1;
        uint32  sequence_num      = 2;
        uint64  uptime_ms         = 3;
        float   cpu_load_percent  = 4;
        bool    is_leader         = 5;
        uint64  timestamp_ms      = 6;
    }
    // QoS: P1, BEST_EFFORT, 1 Hz

## 5.9 SlotAssignment

message SlotAssignment {
        string  follower_id        = 1;
        Pose2D  offset_in_body     = 2;
        uint32  transition_id      = 3;
        float   transition_time_s  = 4;
        uint32  priority           = 5;
        uint64  valid_from_ms      = 6;
        uint64  timestamp_ms       = 7;
    }
    // QoS: P0, RELIABLE, 이벤트 (대형 전환 시, Hungarian 결과)

## 5.10 SurveillanceSectorAssignment (v1.1 신규)

Leader가 각 follower에게 감시 sector를 할당하는 메시지. 10초 주기 (default) 또는 이벤트 시 즉시 발행.

| 속성 | 값 |
| --- | --- |
| Direction | Leader → 모든 Follower + Hub UGV (broadcast) |
| Topic | /swarm/surveillance/sector_assign |
| QoS | P1, RELIABLE |
| Rate | 10초 (정기) + 이벤트 (대형 전환, 위협 식별) |

message SurveillanceSectorAssignment {
        uint32  sequence              = 1;
        uint32  robot_id              = 2;          // 대상
        float   sector_start_deg      = 3;          // -180 ~ 180 (heading 기준)
        float   sector_end_deg        = 4;
        uint32  valid_period_sec      = 5;          // 기본 10
        string  priority              = 6;          // "primary" | "threat_focus" | "overlap"
        string  mode_hint             = 7;          // "sweep" | "track" | "fixed"
        uint64  timestamp_ms          = 8;
    }

## 5.11 PanTiltCommand (v1.1 신규)

Leader 또는 운용자가 특정 robot의 pan-tilt를 제어하는 메시지. SurveillanceSectorAssignment를 받은 follower가 자체 발행도 가능 (sweep 시작용).

| 속성 | 값 |
| --- | --- |
| Direction | Leader 또는 Tablet → 지정 Robot |
| Topic | /{robot_id}/cmd/pantilt |
| QoS | P1, RELIABLE |
| Rate | 이벤트 또는 sweep 진입 시 |

message PanTiltCommand {
        uint32  sequence            = 1;
        uint32  robot_id            = 2;
        float   target_pan_deg      = 3;          // heading 기준
        float   target_tilt_deg     = 4;
        float   speed_dps           = 5;           // degrees/sec
        string  mode                = 6;           // "sweep" | "track" | "fixed" | "engage"
        float   sweep_range_deg     = 7;           // sweep 시 좌우 폭
        uint64  timestamp_ms        = 8;
    }

## 5.12 SwarmHealthSummary

message SwarmHealthSummary {
        uint32  total_robots          = 1;
        uint32  online_robots         = 2;
        float   avg_battery_percent   = 3;
        float   min_battery_percent   = 4;
        float   avg_formation_err_m   = 5;
        float   comm_quality_percent  = 6;
        uint32  robots_in_t4          = 7;
        uint32  robots_in_emergency   = 8;
        uint32  last_received_command_id = 9;   // ★ v1.2: aggregate echo
        uint64  timestamp_ms          = 10;
    }
    // QoS: P1, BEST_EFFORT, 1 Hz, Hub → Tablet

## 5.13 CostMapUpdate (v1.3 신설)

각 UGV가 자체 SBC에서 산출한 Local Cost Map을 publish. Hub UGV의 SBC #1이 cross-validation용으로 수집.

| 속성 | 값 |
| --- | --- |
| 방향 | 각 UGV → broadcast |
| Topic | /{robot_id}/local/cost_map |
| QoS | P2, BEST_EFFORT, KEEP_LAST(1) |
| 발행 주기 | 1 Hz (KPP: cost_map_latency_p99 ≤ 5s) |

## 5.14 LTERoleAnnouncement (v1.3 신설)

LTE 게이트웨이 승격/강등 broadcast. Hub UGV (S2) LTE 작동 불능 시 Follower S3 (1순위)가 자동 승격.

| 속성 | 값 |
| --- | --- |
| 방향 | LTE backup 승격/강등 robot → broadcast |
| Topic | /swarm/lte/role_announce |
| QoS | P0, RELIABLE, KEEP_ALL |
| 발행 주기 | 이벤트 (승격·강등·재확인) |

# 6. 인프라/시스템 메시지

## 6.1 TopicCatalog

message TopicCatalog {
        string  robot_id           = 1;
        repeated TopicEndpoint  publishers   = 2;
        repeated TopicEndpoint  subscribers  = 3;
        uint64  generated_at_ms    = 4;
    }
    message TopicEndpoint {
        string  topic_name      = 1;
        string  message_type    = 2;
        string  qos_profile     = 3;
    }
    // QoS: P1, RELIABLE, TRANSIENT_LOCAL, 이벤트 (시동/변경)

## 6.2 VersionNegotiation

message VersionNegotiation {
        string         peer_id             = 1;
        uint32         schema_major        = 2;
        uint32         schema_minor        = 3;
        uint32         sw_version          = 4;
        repeated uint32 supported_majors   = 5;
        bool           is_compatible       = 6;
        string         error_message       = 7;
        uint64         timestamp_ms        = 8;
    }
    // QoS: P0, RELIABLE

## 6.3 SecurityHandshake

message SecurityHandshake {
        string  peer_id              = 1;
        bytes   certificate_chain    = 2;   // X.509 v3
        bytes   nonce                = 3;
        bytes   signature            = 4;   // RSA 4096
        string  cipher_suite         = 5;   // "AES-256-GCM"
        uint64  timestamp_ms         = 6;
    }
    // QoS: P0, RELIABLE

## 6.4 LogEvent

message LogEvent {
        string  source_id        = 1;
        enum Severity {
            DEBUG=0; INFO=1; WARN=2; ERROR=3; FATAL=4;
        }
        Severity  severity         = 2;
        string  component        = 3;
        string  message          = 4;
        string  correlation_id   = 5;
        uint64  timestamp_ms     = 6;
    }
    // QoS: P3, BEST_EFFORT

## 6.5 TimeSync

message TimeSync {
        uint64  master_time_us     = 1;
        int64   offset_us          = 2;
        float   drift_ppm          = 3;
        uint32  sync_round         = 4;
    }
    // QoS: P0, BEST_EFFORT, 1 Hz, Hub master → 모든 robot
    // PTP IEEE-1588 grandmaster on Hub UGV SBC #1

# 7. 보안 (DDS-Security)

| 기능 | 표준 | 본 사업 채택 |
| --- | --- | --- |
| Authentication | OMG DDS-Security AuthN | PKI X.509 v3, RSA 4096 |
| Encryption | OMG DDS-Security Crypto | AES-256-GCM |
| Access Control | OMG DDS-Security AccessCtrl | Permissions Document |
| Logging | OMG DDS-Security Logging | Hub UGV 집중 저장 |
| Key Management | 사내 정책 | Hub UGV TPM, 임무별 갱신 |

## 7.1 영상 스트림 보안

- SRT 내장 AES-128 암호화 (passphrase 임무별 발급)

- Hub-Operator 구간: IPsec 또는 OpenVPN tunnel (추가)

- Robot-Hub LAN: WPA3-SAE + DDS Security

## 7.2 비상 시 키 폐기

Robot 분실·노획 의심 시 운용병사 앱의 「Compromise」 버튼으로 즉시 해당 robot 인증서 폐기. CRL이 즉시 전체 군집에 broadcast.

# 8. 부록 — 메시지 총람표 (35종)

## 8.1 명령 메시지 (9종)

| # | Name | Topic | QoS | Rate | v1.1 |
| --- | --- | --- | --- | --- | --- |
| 3.1 | FormationCommand | /leader/cmd/formation | P0 | evt |  |
| 3.2 | MissionStateCommand | /leader/cmd/mission_state | P0 | evt |  |
| 3.3 | FireAuthorization | /leader/cmd/fire_auth | P0 | evt |  |
| 3.4 | JammingCommand | /leader/cmd/jamming | P0 | evt |  |
| 3.5 | WaypointCommand | /leader/cmd/waypoint | P1 | evt |  |
| 3.6 | ManualOverrideCommand | /{robot}/cmd/manual_override | P0 | evt |  |
| 3.7 | EmergencyStop | /swarm/emergency_stop | P0 | evt |  |
| 3.8 | OperatorHeartbeat | /leader/heartbeat/operator | P1 | 1 Hz |  |
| 3.9 | VideoStreamRequest | /tablet/cmd/video_request | P1 | evt | ★ 신규 |

## 8.2 텔레메트리 메시지 (9종)

| # | Name | Topic | QoS | Rate | v1.1 |
| --- | --- | --- | --- | --- | --- |
| 4.1 | RobotStatus | /{robot}/status/robot | P1 | 1 Hz |  |
| 4.2 | FormationStatus | /leader/status/formation | P1 | 2 Hz |  |
| 4.3 | DetectedObject | /{robot}/detected/object | P0 | evt |  |
| 4.4 | ThreatAlert | /leader/alert/threat | P0 | evt |  |
| 4.5 | TierStatusChange | /{robot}/status/tier_change | P0 | evt |  |
| 4.6 | FireResult | /{robot}/status/fire_result | P0 | evt |  |
| 4.7 | BatteryWarning | /{robot}/warn/battery | P0 | evt |  |
| 4.8 | VideoStreamHandle | /leader/status/video_handle | P1 | evt | ★ 수정 |
| 4.9 | SLAMUpdate | /hub/slam/global_map_delta | P2 | 0.5 Hz |  |

## 8.3 로봇 간 메시지 (12종)

| # | Name | Topic | QoS | Rate | v1.1 |
| --- | --- | --- | --- | --- | --- |
| 5.1 | FollowerTargetMessage | /swarm/predict/follower_target | P0 | 10 Hz |  |
| 5.2 | BreadcrumbBroadcast | /swarm/breadcrumb | P1 | 0.5-2 Hz |  |
| 5.3 | LeaderElection | /swarm/leader_election | P0 | evt |  |
| 5.4 | EmergencyAlert | /swarm/alert/emergency | P0 | evt |  |
| 5.5 | SLAMLocalDelta | /{robot}/slam/local_delta | P2 | 5초 (v1.3 단축) | ★ 수정 |
| 5.6 | SLAMAggregatedMap | /hub/slam/aggregated_map | P2 | 5초 (v1.3 단축) | ★ 신규 |
| 5.7 | GlobalMapBroadcast | /swarm/slam/global_map | P2 | 0.5 Hz |  |
| 5.8 | HeartBeat | /{robot}/heartbeat | P1 | 1 Hz |  |
| 5.9 | SlotAssignment | /{follower}/cmd/slot_assignment | P0 | evt |  |
| 5.10 | SurveillanceSectorAssignment | /swarm/surveillance/sector_assign | P1 | 10 s | ★ 신규 |
| 5.11 | PanTiltCommand | /{robot}/cmd/pantilt | P1 | evt | ★ 신규 |
| 5.12 | SwarmHealthSummary | /hub/health/swarm_summary | P1 | 1 Hz |  |

## 8.4 인프라/시스템 메시지 (5종)

| # | Name | Topic | QoS | Rate | v1.1 |
| --- | --- | --- | --- | --- | --- |
| 6.1 | TopicCatalog | /{robot}/system/topic_catalog | P1 | evt |  |
| 6.2 | VersionNegotiation | /swarm/system/version_neg | P0 | evt |  |
| 6.3 | SecurityHandshake | /swarm/system/security_handshake | P0 | evt |  |
| 6.4 | LogEvent | /{robot}/system/log | P3 | evt |  |
| 6.5 | TimeSync | /swarm/system/time_sync | P0 | 1 Hz |  |

## 8.5 v1.0 → v1.1 → v1.2 변경 요약

| 버전 변경 | 메시지 | 근거 |
| --- | --- | --- |
| v1.1 신규 (+4) | VideoStreamRequest, SurveillanceSectorAssignment, PanTiltCommand, SLAMAggregatedMap | v1.1 4개 변경 |
| v1.1 수정 (2) | VideoStreamHandle (SRT), SLAMLocalDelta (5초 (v1.3 단축)) | 프로토콜 + 주기 조정 |
| ★ v1.2 필드 추가 (8) | FormationCommand, MissionStateCommand, FireAuthorization, JammingCommand, WaypointCommand, ManualOverrideCommand, EmergencyStop, VideoStreamRequest | A3: command_id (uint32) 추가 |
| ★ v1.2 필드 추가 (2) | RobotStatus, SwarmHealthSummary | A3: last_received_command_id echo |
| ★ v1.2 필드 추가 (1) | LeaderElection | D2: is_hub_ugv + chain_index |
| 유지 (24) | 나머지 모든 메시지 | — |
| 총합 | 35 메시지 (개수 동일, 필드 보강) | schema minor bump |

## 8.6 command_id Echo Pattern (A3)

v1.2부터 주요 명령 메시지 8종에 command_id 필드가 추가되어 ack 메시지 없이도 명령 처리 확인 가능:

- 앱이 명령 발행 시 command_id = uuid_short() 발급

- 로봇이 명령 처리 후 다음 RobotStatus 또는 SwarmHealthSummary에 last_received_command_id로 echo

- 앱이 자기가 보낸 command_id가 echo되면 처리 완료 인지

- 타임아웃 (5초) 이내 echo 없으면 재전송 또는 사용자 경고

**이점**: 별도 ack 메시지 토픽 불필요, 기존 telemetry stream에 piggyback. 고지연·고손실 환경(LTE backhaul)에서 사용자 경험 향상.